Click on Help, then About at top of Main Screen to see Version #

--------------- Mar 18, 2025 Version 4.3 A.052  --------------------

When importing cam files like S96, the program now keeps more

significant figures.  This typically only makes a difference

when importing files created by cam design programs, not from

files measure on actual camshafts.

When importing cam files like S96, the program now keeps more

very low lift data points, down to .00002.  This eliminates

the acceleration and jerk data from having spikes at the

very beginning and end of the opening and closing ramps.

--------------- Jan 8, 2025 Version 4.3 A.051  --------------------

Fixed bug where the program did not always graph lobes for all the

cylinders.  This was a bug we think got introduced in v4.3 A.037

or later.

--------------- Jan 6, 2025 Version 4.3 A.050  --------------------

Fixed bug where results could be blanked out if you opened an old

test that was saved in inches, but program was now set to mm,

or vice versa.

--------------- Nov 30, 2024 Version 4.3 A.049  --------------------

Program is now better at drawing rocker arm on OHC rocker arm

layout screen.

Program is now better at drawing Valve End Pivot Angle on OHC

rocker arm layout screen.

Updated main screen help to now direct you to YouTube videos

rather than videos on performancetrends.com website.

--------------- June 13, 2024 Version 4.3 A.048  --------------------

Fixed minor bug where a dropdown field in the Export Cam Dr. was not

completely visible.

Fixed bug where if you chose to Add Lobe (like from an S96 file)

to an existing file, the specs for the original lobe like

centerline, lash, etc got erased.

Fixed bug where the setting "Exhaust matches Intake" in the

Virtual Follower specs screen was not universally saved

for languaguages other than English.

Increased the limit for doing the Virtual Follower math

for very large Crown Radius inputs for Virtual Flat Tappet

followers.  This did not affect the simulation significantly

because the Flat Tappet simulation at the origial limit

is almost the same as when pushed to the new, larger limit.

--------------- May 15, 2024 Version 4.3 A.047  --------------------

Made probe radius for Virtual Follower much larger for Cam Grinder

so you can determine actual cam profile for Flat Tappet cams.

Now program does not allow blank settings for special Graph types

which would cause problems making graphs.

Program now gives better message when there are jumpy readings in

the region of the measurement to determine cam timing.

--------------- Apr 23, 2024 Version 4.3 A.046  --------------------

Restored a fix from v4.3 A.044 to correctly calculate base circle

effects if exh is different from intake.

Tried to fix bug where electrical noise or other problems would

cause the USB logger to lock up.

Now the software should stop advancing to the next lobe if you are

on the last lobe no matter how you have Preferences set.

Fixed bug where Jerk was reported with the wrong sign, + and -.

--------------- Feb 17, 2024 Version 4.3 A.045  --------------------

Fixed bug where the Virtual Cam screen would not always be updated

correctly for the OHC rocker arm layouts when you changed Base Circle.

Program now shows the labels above the graph on the main screen of

Centerline, Duration, Lift, etc. for cams "Measured with Electronics"

larger, to better ensure all the data is displayed on higher

resolution computer screens.

--------------- Dec 20, 2023 Version 4.3 A.043  --------------------

Fixed more bugs where errors could occur (even without error messages)

when working in certain languages other than English.

--------------- Dec 19, 2023 Version 4.3 A.042  --------------------

Made further refinements to Exporting S96 files.

Fixed bug where a "DrawTabsError" error message was produced if you

were working in certain languages other than English.

--------------- Dec 18, 2023 Version 4.3 A.041  --------------------

Made further refinements to Exporting S96 files.

--------------- Dec 16, 2023 Version 4.3 A.040  --------------------

Changed program so the recording screen should always be on top

of other screens.

Made changes to the Virtual Follower simulation when the measurements

were not started well centered on base circle, especially for OHC

Rocker Arm valve trains.  Data at very low lifts could have "jumps"

in the simulated lift of approximately .005" lift or more depending

on where the measurement was started.

Fixed minor bug where if you clicked on one of the "Export" options

the mouse pointer would turn into the hourglass instead of stay

as the arrow.

--------------- Nov 23, 2023 Version 4.3 A.039  --------------------

In the Checking Limits report, the Lobe Separation label now is reported

as Lobe Separation.  Previously it was Exh Lobe Separation.

Added Label to Virtual Follower screen for OHC rocker arm layout to

show if CW or CCW was chosen for 'Rotation' in the Cam Layout Details

screen, to explain why the view says Viewed from Front or Back of Engine.

Fixed bug in Virtual Follower screen where the Exhaust was not always

drawn correctly (could be drawn same as the Intake).

--------------- Nov 2, 2023 Version 4.3 A.038  --------------------

Fixed bug where the Checking Limits report did not work for all options

in the Plus version, but did in the Cam Grinder version.  Now it works

in both versions.

Fixed bug report data could be strange if durations were asked for

at a lift greater than the lobes max lift.

--------------- Nov 1, 2023 Version 4.3 A.037  --------------------

Program should now be better at keeping a backup config file if

config file gets corrupted.

Fixed an additional bug where reports were not being done correctly,

especially if the first cylinder number on the cam was NOT 1.

Tried to fix bug where electrical noise or other problems would

cause the USB logger to lock up.

Now the software should stop advancing to the next lobe if you are

on the last lobe no matter how you have Preferences set.

--------------- Oct 28, 2023 Version 4.3 A.036  --------------------

Fixed bug where the Radius of Curvature was being calculated on

the raw, measured data.  Now it is calculated based on the actual

cam lobe, as if measured with a knife edge follower.

Fixed bug introduced in v4.3 A.028 where the "Compare Lobes" report

was not working correctly.

Fixed some bugs where reports were not being done correctly, especially

if the first cylinder number on the cam was NOT #1.

--------------- July 3, 2023 Version 4.3 A.035  --------------------

Program now gives better messages for turning the cam too fast

for motorized stands.

Program now warns you if you have asked to specify custom centerlines

for a Cam Card but the centerlines entered are VERY unusual or blank.

--------------- Apr 24, 2023 Version 4.3 A.033  --------------------

Fixed bug displaying correct crank deg numbers when encoder is mounted

on the crank.

--------------- Apr 3, 2023 Version 4.3 A.032  --------------------

Fixed bug reading furlong data rotary encoder.  This could have

introduced rare "dead bands" where data was not recorded for 2-3

deg of rotation.

Added more files to list of acceptable Example cams to be open in

demo mode.

Now check marks are updated when you change the Legend Text Size in

the Graphs screen.

Program gives better error message for a measurement when the starting

lift and ending lift are not close together (not on base circle).

Graph settings after initial installation are now more typical.

Tried to streamline the messages when opening up the demo version.

--------------- Mar 18, 2023 Version 4.3 A.030  --------------------

Made several changes to better display Cam Degrees on the recording

screen, and Crank Degrees if they have been requested.  Now the

"Degree Wheel" gauge will always show 0-360 degree markings,

UNLESS you have requested Crank Degrees to be displayed.  Then

the "Degree Wheel" gauge will display markings you have requested

in the Test/Cam Setup screen.

--------------- Feb 18, 2023 Version 4.3 A.028  --------------------

Fixed bug where Comparison Reports were not working correctly.

Made some slight modifications to make Comparison Reports look better.

--------------- Feb 16, 2023 Version 4.3 A.027  --------------------

Program now better sets the X scale for Frequencies vs RPM graphs.

Eliminated some messages about needing the profile to be measured

as a Virtual Follower for some analysis when this was not required.

When converting a CPP file, the program now better steps  you

through the process with a message in the Test/Cam Setup screen.

When doing a conversion of a cam file to "Measured with Electronics",

the program now explains that for most conversions you do NOT want

to include the Base Circle measurement.

When doing a conversion of a cam file to "Measured with Electronics",

fixed a bug where other settings in the program could prevent this

from happening correctly.

Now if you manually set the Graph Scales, you can set the X axis to

a very large number, like if you are plotting Frequencies vs RPM.

Now if you manually set the Graph Scales, the program does not check

that you have enter valid X and Y limits if you have selected to do

Auto Scaling.

Now the program assigns any Valve Lift less than 0 as 0 lift.  Cam

Lift values less than 0 (like runout) should not be affected.

--------------- Dec 30, 2022 Version 4.3 A.026  --------------------

Fix bug when doing graphs as viewed from the end of the cam (circular)

introduced in v4.3 A.016 to refresh grid

Fixed bug introduced v4.3 A.016 where the "Limit Line" for FFT

graphs was not being shown on the graph.

Fixed bug where the Valve Acceleration was not being done correctly

at lifts below valve lash.

Fixed some bugs where making "Pick from List" graphs.

Fixed bug where if the camshaft turning direction was CCW and you

refreshed the virtual follower cam profile, the data could be incorrect.

Increased some limits inside the code to allow more agressive Virtual

Follower OHC rocker arm geometries to be calculated.  Previously

it just produced an error message.

Fixed bug where "End View" graphs were not always shown with

the best scale so circular portions of the profile looked circular

and not oval.

Fixed bug so you can exit the Test/Cam Setup screen when trying to

undo Virtual Follower changes.

--------------- Nov 19, 2022 Version 4.3 A.025  --------------------

Added feature to eliminate some error messages which were not needed.

If you select the Option of "Cam Lobe may have 2 bumps", the program

now also looks for a flat closing ramp significantly above base circle.

Updated Example Layouts43.txt to put the rotary encoder on the Rear

of the Briggs Flathead layout, which is more typical.

Fixed bug for Cam Grinder version if doing Cam Grinder timing where

the labels on the main screen graph could be wrong.

Added label to lower left corner of graph to show whether graph

is for Crank or Cam Degrees.

--------------- Oct 4, 2022 Version 4.3 A.024  --------------------

Fixed bug where Graph Name in History Log was not being updated

correctly to reflect file name.

The program now handles the situation when you have opened

a file with a 'Cam Design' which is not in the list of

"Cam Designs" in your program.  This occurs when someone with

an newer version or their own 'User' Cam Design sends you

a cam file.

Now able to install USB driver installation for Tester screen

under Tools

Program is now much better at finding problems with the

"Lobe Description" in the Cam Layout screen and warning

you of possible problems.

Program now better checks the intake and exhaust lobes possible

for doing a graph of all data files to be graphed, and allocates

the proper graph lines.  Before this, the program took the pattern

of the current test open on the Main Screen.

--------------- Feb 12, 2022 Version 4.3 A.021  --------------------

Added new Preference to allow specifying lower lash points

to be allowed in Test/Cam Setup screen, Cam Grinder only.

Fixed Help "balloons" over some buttons.

Enlarged EZ Data Entry for manual data entry.

Improved/updated help screens for EZ Data Entry for manual data entry.

Fixed bug where comparison reports of 2 or more files may not make

correct comparisons if the Degree Wheel Type for the different files

were different.

Program now makes sure rows for doing manual data entry are enlarged

to accept data.

Got rid of message about the electronics if you are starting a new test

for manual data entry.

Help "balloon" and help caption for Centerline now shows the method

for determining centerline, based on a Preference setting.  NOTE:  It

is only done for tests recorded or recalculated after this version,

because files from earlier versions were not identified as to how

CL was calculated.

Now in Report Screen, if you switch between Single and Multiple

tests, the report updates accordingly.

Made several changes in History Log screen to identify if you are

using the History Log to make Graphs or Reports.

Now when you make changes in the History Log, the Report screen

will be updated with your requests.

--------------- Oct 8, 2021 Version 4.3 A.020  --------------------

Now if you are displaying the Crank Degrees while you are measuring

a lobe, the Crank Degrees will better match whatever type of Deg Wheel

you have selected in Test/Cam Setup.

Now the program stops saying "Updating Display Only (not recording)"

after you are actually recording data.

Updated the Help files for Special Report Options and Virtual Follower

screens.

Program now warns if you use Preference to advance to next lobe going

to the right on the cam (Cam Position Preference), but you just measured

the rightmost lobe, you are at the end of the cam.

Fixed bug where a Metric conversion was being based on the wrong Preference

setting.

Improved the process for exporting .igs files in Cam Grinder version.

--------------- Sept 21, 2021 Version 4.3 A.019  --------------------

Fixed bug where Valve Spring screen was corrupted, Cam Grinder version

only.

Fixed bug where Virtual Flat in the program was misspelled as Vitual

Flat.

Fixed label in Test/Cam Setup screen where metric options did not show

the name Lift for Rating Events for that input.

--------------- Sept 13, 2021 Version 4.3 A.018  --------------------

Fixed some bugs for displaying Crank Degrees on the Recording screen

and where the digital reading did not always match reading on gauge.

--------------- Aug 4, 2021 Version 4.3 A.017  --------------------

Lowered limit of the Probe Radius for doing Virtual Follower down to

0.01 inches and 0.25 mm.

Fixed help file for the Probe Radius which used the word "diameter" when

it should have said "radius".

--------------- Aug 1, 2021 Version 4.3 A.016  --------------------

Fixed bug where the requested grid style was not drawn correctly

the first time a graph was created.

Program now puts check marks by Grid Style being used.

Program now puts check marks by Background Color being used.

Fixed bug where the graph cursor was not drawn in correct location

if the maximum X value was a negative number, like a graph from

-360 to -180 degrees.  This fix could help other graph features where

the maximum X value was a negative number.

Fixed bug where some graphs in Cam Grinder version could get corrupted

if you requested "Int & Exh" but had only Int or Exh profiles.

Eliminated the logger option of "Mini USB w Lift Pot." because we

never released it.

--------------- July 15, 2021 Version 4.3 A.015  --------------------

Program will NOT allow you to "Check Com Port" or "Boot Message" unless

you have the older Black Box II logger.

Updated method of displaying PDF files.

--------------- July 7, 2021 Version 4.3 A.014  --------------------

Eliminated redudant mnuRequestRemoteAssistance under Main Screen Help.

Fixed bug introduced with some recent changes where program

would not create files for Engine Analyzer Pro when requested to

do so from Engine Analyzer Pro.

--------------- Apr 1, 2021 Version 4.3 A.013  --------------------

Fixed bug where you could not choose different loggers in the

Tester Calibration screen in the Basic Version.

--------------- Feb 26, 2021 Version 4.3 A.012  --------------------

Improved method of exporting some data file types which produced files

for CNC machining.  Before this the export process left a gap

when the 2 ends of the data came around to each other, typically

on base circle.

Fixed bug where results from exporting various data files could

produce inconsistent results, depending on program features used

before exporting.

If using a potentiometer for lift, the program now does some

additional smoothing of the data.

When exporting an S96 file, the program produces a better file

if the first data point is not exactly at 0 lift.

--------------- Oct 26, 2020 Version 4.3 A.011  --------------------

Added new "Electronics" option of "Mini USB w Encoder" and

"Mini USB w Pot."  This is the new logger to replace the older

Black Box II.

Program is now better at updating the History Log when you open a

file, but also chose to save changes to the previous file.

Lash is now saved to 3 decimal places.

--------------- Oct 27, 2019 Version 4.3 A.010  --------------------

Trying something to better ensure Reg Code and Config settings are

saved correctly.

If you enter an improper File or Folder name in the EZ Start Wizard,

now you are warned to correct this right at that screen, and not

wait until the Wizard wants to start a test.

Improved some wording in the EZ Start Wizard.

Fixed some errors which would produce messages after recording some

data if the data looked somehow wrong.  This would cause the program

to quit with an error 402 message.

--------------- June 29, 2019 Version 4.3 A.009  --------------------

Made several enhancements to "Lobe Analysis" report when using

millimeters for lift.  Report now also explains why the "Radius

of Curv." calculations are blank if not using "Virtual Follower".

Fixed a bug where if you had a cam with just 1 intake and 1 exhaust

lobe, the program may not always make reports of both lobes

if that is what you requested.

Fixed a bug where if you converted a profile to 'Measured by Hand'

or 'Measured w Electronics', it could be markekd that the Rotary

Encoder had never been indexed to the cam.  This typically did not

matter unless you wanted to advance or retard the cam after making

this conversion.

Refined messages when searching for Com Port for Cam Pro Plus.

--------------- June 10, 2019 Version 4.3 A.008  --------------------

Changed "Low Lobe Area" to "High Lobe Area" and vice versa because

these were being labeled wrong.

Fixed bug where the "Advanced (browse)" in the "Save As" form was

missing.

Fixed bug where rezeroing rotary sensor did not always work.

Changed some labels from Cam Analyzer v4.0 to Cam Analyzer v4.3

--------------- May 16, 2019 Version 4.3 A.007  --------------------

Added Straight-up.txt help file to installer which explains

how to Make Centerlines Equal under Edit on main screen.

Made some enhancements to the "Save As" screen.

Fixed bug where proper file name was not highlighted in the

"Save As" screen by default.

Program now does not produce additional Cam Dr files .msu and

.des when producing some more advanced file types which are

not Cam Dr format.

The program is now better at knowing if you have made critical changes to

Lifter Bore Angle Details and Cam Layout Details to give warnings

about being sure you want to make the change.

--------------- Apr 22, 2019 Version 4.3 A.006  --------------------

Fixed bug where program would not let you save a file to

the first folder in the list in the 'Save As' screen.

Fixed minor bug where program would explain why settings were not

saved when you Cancelled starting a New Test, even when you

actually did start a new test.

Made program more likely to find Adobe printers.

Fixed minor bug to prevent program getting into an endless loop

if the first settings in the Calculation Utility screen were outside

the allowed limits.

In "Export as Cam Dr..." under File, the Cam Grinder version now

has the option of "Cam Checker" as a file format.

Added CamDrB.clc file to installer.  Seems to have gotten dropped some

how.  This allowed for more advanced types of exporting of graph files.

When opening a CPP file, the program now goes through the Test/Cam Setup

screen so cam timing and centerlines are displayed correctly.

Added Cam Layout for "Ford 1.6L 4 cyl pushrod Kent" to Example Layouts43.txt

Fixed bug where some Graph Data exported under File could have slightly wrong

data written for 1 data point when the X data was exactly .00000 when doing

graphs looking from end view of cam (Cam Grinder only).

Fixed minor bug in Cam Grider version when you choose to "Pick from List" data

to graph, certain parameters pertaining to this graph (listed under the "List"

were not immediately enabled until you clicked on something else.

--------------- Feb 15, 2019 Version 4.3 A.005  --------------------

Program now stops printing if you Cancel when printing a graph

to a PDF printer.

Updated caption on Graph screen to v4.3.

Added new 'Look for New Adobe Acrobat Reader' button to Preferences

to look for an Adobe Acrobate Reader file for displaying

PDF help files, like the User's Manual.

Updated Preferences Help file.

Program now better ensures wrong graph labels are not

held over from a previous graph if something goes wrong

with displaying graph lines.

Program is now better at displaying graph lines for overlay

graphs when settings do not make good sense, like when asking

for Intake and Exhaust lobes to be graphed, but the files only

have Intake lobe data.

Adjusted spacing to allow more space for 3rd column in printed

headers for reports and graphs.

Now if you ask for graphs of "Absolute Lift" (including base

circle offset), but you did not measure the cam with Absolute

Lift, the program uses the Base Circle entry in Virtual Follower

screen as the base circle.  Cam Grinder version only.

--------------- Nov 18, 2018 Version 4.3 A.004  --------------------

Fixed possible problems doing some Virtual Follower conversions.

Fixed possible bug where converting a data file to 'Measured w

Electronics' if the closing ramp was very close to the end of

the recorded data.

Added a new Preference to check for 'bumps' created during a Virtual

Follower conversion.  This Preference under the Calculations tab is

for 'Fix 'Bumps' from Virtual Follower Conversion'.

Added Intro Help screen to explain what your options are if a

'bump' is encountered during Virtual Follower Conversion.

--------------- Nov 1, 2018 Version 4.3 A.003  --------------------

The program could 'bomb out' on some operating systems when looking

for the default printer, for restoring it when shutting down.  Now

this is sensed and fixed next time you start the program.

Changed all references of "Master Encoder" to "Master Cam Tester" in

Cam Tester specs screen.

Updated some Combo43 and Text files to installer which produced

error messages when going into the Test/Cam Setup screen.

--------------- Oct 15, 2018 Version 4.3 A.002  --------------------

Program now has updated and proper version of User's Manual PDF and

v4.3 Features PDF files.

Program now has a new Preference to let you manually load the path

to a proper version of Acrobat for displaying PDF files.

Message describing what happens in Acrobat now shows message of

this new Preference and how to use it if Acrobat does not open.

*******************************************************************

******************  v4.3 A.001  Oct 14, 2018  *********************

*******************************************************************

New Features for the "Cam Grinder" Version Only:

Weve added a feature to allow for a Test Piece picture to be included

with your test files and printouts.  Program now does not print pictures

if either the Logo or Test Piece picture file does not exist.

Cam Grinder Version has new options where you can create a Cam Card

from separate intake and exhaust files in the special Cam Grinder T

iming format.  Cam Grinder Timing files were added in v4.0 and do not

force the centerlines to line up from different cylinders.  They do not

account for lifter bore angles or firing order effects.  Instead the

profiles look much as they would when looking at the end of the cam.

New Graph and Report option for Cam Grinder, allow for Crank Degrees

or Cam Degrees.   If doing Cam Degrees, the program turns Off the Labels

of TDC and BDC and always numbers the degrees in reports as numbers,

not as a deg wheel.

In Cam Grinder version, added a new Rezero Rotary Sensor Option in the

Record screen.

In Cam Grinder version, added a new Rezero Lift Sensor Option in the

Record screen to allow you to specify a certain lift value to be for

the lift sensor at it's current position.

Added a new profile type for "Measure with Electronics" of "Measure

Anything", for cams which are not typical automotive cams.

Added a new Graph Type for the Cam Grinder called Overlap Area.

It can be done for either Valve Lift or Tappet Lift.  You must also

include both Intake and Exhaust Valve or Tappet Lift graphs for this

graph to be produced.

Program now allows for .csv types of output formats and Blair file

formats for Exporting.

Crank Degrees can now be shown on the Electronics Recording screen

in addition to Cam Degrees.

Program has several new features for Exporting Manufacturing Files, including:

Added special feature for exporting a Master Cam file.  Fig 9.7.

Added option to export Manufacturing Type file in IGES or IGS format.

Then you can also specify the base circle to .igs files.

Added feature to allow user to zero out runout on measured profile.

If you set this to Yes, the program will find what the program believes

to be the start of the opening ramp and end of the closing ramp.  Then

whichever lift is higher will be assigned the lift of the base circle

of the master cam to be created.  This will eliminate any runout of

the measured profile being passed on to the master cam.  The 'downside'

of doing this is that if there is a significant difference between lifts

at these 2 points (bad runout on measured cam), there could be a discontinuity

at start or end of one of the ramps.

Added feature to increase or reduce the lobe durations (stretch or compress

duration) by a percentage.

Cam Grinder version lets you manually generate degrees up to 720 degrees.

This can be handy for converting cam files Measured with Electronics to

Measured by Hand so you can do modifications on them.

The program now allows much larger Roller Follower diameters, to simulate

grinding stones on cam grinders.

If this is set, duration and events are done in Cam Degrees, the Degree Wheel

data on main screen is now labeled "Cam Degrees" if doing Cam Grinder timing.

Added the ability to read .P files.  Click on File, then Open from All Saved

Tests and click on a .P file to open it.

New Features for the Plus Version Only:

There is now a Preference setting to let you keep the Graph on the main screen

always at the same graph scales.  This can cause problems if you open files

for cams with different lifts and durations than you normally run, or the cams

you test produce quite different lifts and durations.  However, if your cams

are always about the lift and duration, this new Preference makes it easier to

spot changes with the graph on the main screen.

Program now states events on the Cam Card are at valve lift if you are using

virtual follower rocker arm "Cam on Rocker".

If you are creating a Cam Card from a file which contains only intake or only

exhaust data, but you have already picked a file to be associated with this

file for the other lobes data, the program now reminds you of the associated

file when you create a cam card.  You can also choose to change or cancel the

file associated with this file.

A new Overlap option has been added to the Custom Duration Report.

Two major new Options screen have been added for the Cam Card, including when

clicking on the Options button at the lower right:  More Options and

odify Labels.

The program now displays a graph title on the main screen, stating if the

profile is measured with the actual follower, or simulated by Virtual Follower

and what type of Virtual Follower.

The program now saves the current file as a temporary file before making a

Cam Card, so that any changes made during creation of the Cam Card can be

restored when program returns to the main screen.

The program now allows a roller follower to be one tenth (0.1) the normal

lower limit, which is nearly a "knife edge" for the Virtual Follower

simulation.  This way the Cam Lift numbers can be very close to those measured

by the actual pointer, or even smaller.

The powerful Filter feature lets you search for files containing particular

measurements, or comments, etc.  (See Appendix 8 for more info on this

feature.)  Now the list of files fitting a particular Filter criteria are

displayed in Notepad with better spacing to allow for very long file and folder

names when you click "Print list of all files fitting these conditions".  This

list now also includes the Cam Number.

New Features for All Versions:

You can now request what to use for "Lift for Rating Events", either .050",

.040" (1 mm) or .053" (for Harley Davidson).  Added Rated Lift to History

Log, and better arranged spacing of History Log columns for information they

must contain.

The new Mini USB is a possible logger type.  Right now this is just a direct

replacement of the current Black Box II.  In the future we may add more features.

Now if you import a cam profile, and the profile has 0.5 deg increments and

more than 400 rows of data, the program checks to see if the first row has a

whole degree increment like 0.0, 1.0, 2.0, etc or half degree 0.5, 1.5, etc.

The program will use the rows where there are whole degree increments.

Replaced "Copy" with "Copy (or Merge)" to list of Folder Options possible in

'Save As' screen.

Program now explains why changes in 'Starting a New Test' screen are not saved

if you don't start a new test.

Added features so program can check after writing the config file if it is

corrupt and whether a backup should be made.

A backup file of the config file is now made and used if the main config file

has been corrupted.

Form for Starting a New Test now has larger fields for File, Folder, etc

names to better display the names completely.

Program now saves tests to My-Tests folder if it's original folder was the

Examples folder.  Overwriting a file from the Examples folder is not allowed.

If you change the printer within the program to something other than the

computer's default printer, the program now restores the default printer

and printer orientation when it shuts down.

Program now adjust length of folder and file name in History Log to better

fit in available space in the 1st column.

You can now import files from the Performance Trends Quick Cam Checker.

You need v1.1 A.015 or later of the Cam Checker, which should be a free

download on the PerformanceTrends.com website under Downloads.  NOTE:

The precision of these files are not as good as those measured with the

Cam Analyzer on our Cam Test Stand.

Added several new features to the Save As screen so you can see other

files in folders and select these file names as the name to use or edit

to use as a new file name.  It also has simplified the method of creating

a new folder, and listing these file names by date or name.

Now the browser should be the default on your computer.  Previously it

only looked for Internet Explorer.

When you resize the main screen, it is more reliable at refreshing

all controls and pictures.

Program can now better display PDFs in some versions of Windows 10.

Did lots of refinements to the buttons on the Graph screen which let you

enlarge or shrink the graph, or shift the graphs left, right, up and down.

Prior to this, the action could get "stuck" or not do the action exactly

as expected.

Program has several refinements for reading CPP files more accurately

and reliably.

Added new Cam Layout types of "Hemi 99" for race only Mopar, the modern

Mopar Hemi 5.7/6.1/6.4L, the GM Gen V LT-4 2015, and fixed a bug in

Pontiac cam design layout.  Cam Layout types are used for data files

types of Measuring with Electronics.

Program now reports significant figures for lift to 5 decimal places.

The measurement resolution of lift sensor is .00004 inches, so going

to more than 5 decimal places does not produce any more accurate results.

Program can now read SVL files (from 4stHEAD program).

Program now just displays "Hydraulic" or "Solid" for the Lifter Type for

cams that are Measured with Electronics.  Previously you could specify

various roller or flat Lifter Types, but that did not make sense for cams

that are Measured with Electronics.

Replaced linear interpolation with curve fitting for exporting cam files

for all versions.

Files and folders you delete now are actually sent to the Recycle Bin so

they can be recovered later if you want.

Program is now better at adding numbers to the end of file names by default.

Now when you open a file which has a Graph Name specified in the History

Log, that Graph Name stays with the file.  Previously it was restored to

the default File Name as being the Graph Name.

Program now tells you your specs "MATCH" the master specs.  Before it

said "DO MATCH" and many people mistakenly read this as "DO NOT MATCH".

Fixed minor bug where if you changed the Graph Name for the first file

in the History Log, it would not stay permanently.

Fixed bug where if cam file has only exhaust lobes, you could not open

it from the History Log on the main screen.

Refined the program to be better at reading Black Box II when you have

selected the Preference of 'Assume Asian Operating System'.

When converting files between Inches and MM lift, now the program uses

more decimal places for all files types for more precise conversions.

^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^

^^^^^^^^^^^^^^^^^^^^^  v4.3 A.001  Oct 14, 2018  ^^^^^^^^^^^^^^^^^^

^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^

--------------- July7, 2021 Version 4.0 A.050  --------------------

Fixed bug introduced with some recent changes where program

would not create files for Engine Analyzer Pro when requested to

do so from Engine Analyzer Pro.

--------------- Sept 1, 2018 Version 4.0 A.049  --------------------

Fixed bug where the option of printing the Company Logo graphics

file on a Cam Card was lost, somewhere around v4.0 A.030 or so.

Now that feature is restored.

Fixed bug where if you clicked on a label in the Tester

Calibration screen, the program could bomb.

--------------- Jan 23, 2018 Version 4.0 A.048  --------------------

Fixed bug in Pontiac cam design layout. Modified Example Layouts.txt file.

--------------- Nov 11, 2017 Version 4.0 A.047  --------------------

Program now shows all columns in the history log.  Previously it did

not show some "graph" columns unless it was opened from the Graph screen.

The screen warning that commas are bring used where the program is

expecting decimal points "." is now forced to be on top of other

screens.

Fixed bug where lobes were not always included in the graph if the cam

was all intake or all exhaust lobes.

--------------- Feb 14, 2017 Version 4.0 A.044  --------------------

Fixed Error 480 bug if restarting with corrupted config file.

Fixed bug where the runout could be calculated wrong if you did

not start recording the data somewhat well centered on the base

circle.

Did some other tweeks to the runout calculation if the start of

the opening and closing ramps were not found at somewhat the same

lift.

Fixed bug where program could bomb if you printed main screen to

PDF type printer and then cancelled printing.

Added ability to read .P files for Cam Grinder version only.

--------------- July 29, 2016 Version 4.0 A.042  --------------------

Improved emailing functions.

Program now warns user to set 'Allow Less Secure Apps' to ON

for Gmail accounts to be able to send emails.

--------------- May 12, 2016 Version 4.0 A.041  --------------------

Added Cam Card option of 'Close', to close the Cam Card.  Previously

you had to click the Red X at upper right.

Program now warns you that you can not do a Cam Card starting with

a cam which only consistes of Exhaust Lobes, but how to use this

"exhaust lobes only" cam on a cam card.

Program can now better create cam cards from separate intake

and exhaust files.

Program now saves the current file as a temporary file before making

a cam card, so that any changes made during creation of the cam card

can be restored when program returns to the main screen.

If doing Virtual Rocker Arm (Cam on Rocker Arm) type of follower, the

Cam Card no longer shows the Rocker Ratio.

--------------- May 11, 2016 Version 4.0 A.040  --------------------

Fixed a bug where if there was significant runout on base circle,

the conversion to the actual cam profile (knife edge data) could

have a small spike or dip on the first data point of the conversion.

Program now warns you if you are trying to make a Cam Card from 2

separate files (intake and exhaust lobes) and one file is in

"Cam Grinder" format and the other is in standard format.

Added option on the Cam Card to turn off the association of one file

with another for files which have only either all intake or

all exhaust lobes.

Refined some notes about events measured at Valve Lift when

doing Virtual Cam on Rocker Arm files.

--------------- May 7, 2016 Version 4.0 A.039  --------------------

Fixed bug where if you specified nothing for the Cam Layout

the program could bomb out.

Program now gives more information and help about Cam Layout

specs not being correct.

Program now states events on the Cam Card are at valve lift

if you are using virtual follower rocker arm "Cam on Rocker".

When you start a cam file using the EZ Start Wizard, the program

now explains better that picking the Intake CenterLine, Intake

Lift at TDC, etc are now "Int (or Exhaust) Centerline", etc

and the Exhaust will be used if the cam has only Exhaust lobes.

If you are doing a Virtual Follower Rocker Arm simuilation,

the Rocker Arm "behind the scenes" is set to 1.0 to prevent

any math being incorrect.  That is because the rocker ratio

is being simulated by the Virtual Follower Rocker Arm simuilation.

Added an additional note in Test/Cam Setup screen of "See note

above" to explain why Rocker Ratio is not a visible input.

Fixed bug where Rocker Ratio would not always re-appear if

it was needed as an input.

Fixed bug where the Virtual Follower simulation of a Rocker

Arm follower could produce unusual results, where the resultant

valve lift could be 180 deg out of phase with what you measured

on the tappet.

--------------- April 9, 2016 Version 4.0 A.038  --------------------

Refined program to be better at reading Black Box II when you have

selected the Preference of 'Assume Asian Operating System'.

When converting files between Inches and MM lift, now the program

uses more decimal places for all files types for more precise

conversions.

Did lots of refinements to the buttons on the Graph screen which

let you enlarge or shrink the graph, or shift the graphs left,

right, up and down.  Prior to this, the action could get "stuck"

or not do the action exactly as expected.

Fixed a bug where one angle for either of the OHC Rocker Arm

geometries for the Virtual Follower specs was corrupted when the

program converted files from Metric to English or vice versa.

Fixed bug where some critical specs could be held over from a

previous file if you opened an older Cam Analyzer file, which

had a very different file format.

Fixed bugs for the "Do 'Cam Grinder' Timing" only in this Cam

Grinder file format when doing the Lobe Analysis report.

--------------- Mar 12, 2016 Version 4.0 A.037  --------------------

Fixed bug where some Exporting of Mfg files feature would only export

the first lobe on the cam.

Fixed bug introduced in v4.0 A.032 where the lower limit for

a probe radius for the Cam Grinder version was increased to

too high a value, like 7 mm or .3 inches.

Fixed some bugs for the "Do 'Cam Grinder' Timing" only in this Cam

Grinder file format where the Duration Report and Cam Card has

some errors, because 'some results were reported in Cam Degrees

instead of Crank Deg.

Tried to make the degree increments picked for graphs better

choices for degrees, like 45, 60, 90, 180, etc.

Fixed bug where if a cam had only exhaust lobes and you graphed

it with cams with only intake lobes, the exhaust lobes may

not be graphed.

If you are creating a cam card from a file which contains only

intake or only exhaust data, but you have already picked a file

to be associated with this file for the other lobes data, the

program now reminds you of the associated file when you

create a cam card.

Fixed bug where if you picked an associated file for the other

cam for doing a cam card, the Current File on the main screen

could be changed to the associated file you picked.

--------------- Feb 21, 2016 Version 4.0 A.036  --------------------

If a test name is more than 50 characters, the program suggests a

name of the first 50 characters you have entered.  Previously it

suggested just the first 12 characters.

The Degree Wheel data on main screen is now labeled "Cam Degrees"

if doing Cam Grinder timing.

Added labelto show if linear sensor has been set up to read in the

reverse direction, like when done in the engine (sensor extension

is positive lift).

Label for noting if linear sensor is in Inches or MM is now more

readable on various computer screen resolutions.

Modified the requirements for the program saying you are turning

the cam too fast when doing measurement, to prevent error messages

which are not critical.

Now have label explaining if rotary encoder is on crankshaft

or on some other rotating component.

Changed the scaling on the linear encoder gauge to include

negative readings.

--------------- Feb 15, 2016 Version 4.0 A.035  --------------------

Program now warns that you can not do the 'Analyze' graph feature

if you are graphing 'Show End View of Cam'.

Fixed bug in Start New Test Wizard which would happen if the test

you were using for a 'Pattern' was using a 'User Finds...' option

for finding TDC.

Added a note that if the program mistakenly believes the box has

been reset just because both Lift and Deg are exactly 0.00, that

you can still continue with your measurements.

--------------- Jan 30, 2016 Version 4.0 A.034  --------------------

Fixed bug where finding TDC with the linear encoder was not

always working correctly, depending on direction of rotation

of the cam.

Fixed bug where a graph would not include a data point for 0

degrees, but would include -2 and 2 degrees.

For Cam Grinder layout, now the output is centered around TDC, or

between -180 and 180 degrees.  Previously it was centered between

0 and 360 degrees.

--------------- Jan 12, 2016 Version 4.0 A.033  --------------------

Put in a fix for reading CPP electronics when linear encoder does not

read negative numbers correctly.

Added .csv types of output formats for Exporting, which moved the

Blair type down 2 spots in the list, from 16 to 18.

Fixed bug where if using Millimeters for lift, the Probe Radius

you input for the Measuring Probe in the Virtual Follower

screen would not be correct, unless you chose one of the 2

preloaded standard types.

Fixed some bugs when converting lobes measured in mm to Actual Cam

Profile and simulating other virtual follower types, especially

if you have said you are measuring Absolute Lift.

In Cam Grinder version, the program now allows much larger Roller

Follower diameters, to simulate grinding stones on cam grinders.

Renamed an option for importing data as a cam profile from "Read

Data as Polar Coordinates" to "Read as 2 colums of Degrees and Lift

(polar coordinates)" to be more clear.

Fixed bug where if you tried to import garbage or nothing as a cam

profile, the program could bomb out.

Set the option for importing a cam profile to Crank Degrees for

degrees.  Before neither cam or crank degrees was set so it could

be confusing unless the user picked one.

Now if you import a cam profile, and the profile has 0.5 deg

increments and more than 400 rows of data, the program checks

see if the first row has a whole degree increment like 0.0, 1.0,

2.0, etc or half degree 0.5, 1.5, etc.  The program will use

the rows where there are whole degree increments.

--------------- Oct 5, 2015 Version 4.0 A.028  --------------------

Enlarged size of CPPg&(6) to CPPg&(10) for reading extra bits if

they are read from the CPP electronics.

Added Preference to provide more time when reading the CPP electronics,

under 'Electronics' tab, called 'Slow Down CPP Readings'.  Typically

a setting of 2 or 3 mSec is enough.

Program now gives more testing tips for using the CPP electronics.

Eliminated some references to Flow Benches in the EZ Start Wizard.

--------------- Sept 10, 2015 Version 4.0 A.027  --------------------

Did some refinements of the math to determine TDC location when using

a sensor.

--------------- July 31, 2015 Version 4.0 A.026  --------------------

Fixed bug where Master Tester Specs were not being loaded correctly.

Fixed minor bug in EZ Start Wizard where program would ask about Copying

data from another program even if you said you were going to measure the

cam with electronics.

Program now ALWAYS warns if a gap in measurements is more than 12 Cam Deg,

even if the Preference setting says not to warn.

Program now just looks to see if 2 successive degree readings are equal to

see if the 2nd reading can be disregarded.  Before both the deg and lift

had to be equal.  This helps the program fill in missing data as some

degree increments more accurately.

If program warns you that you can not save a file to the Examples folder

when you exit the program, the program now does NOT continue to shut down,

but returns you to the program so you can save the file to a different folder.

--------------- June 25, 2015 Version 4.0 A.025  --------------------

Fixed very minor bug where a lobe which had not yet been measured could

produce an incorrect error message in Sub FilterLobe.  This did not

introduce any errors in the data, but was just a nuisance.

Cam Grinder version lets you manually generate degrees up to 720 degrees.

Fixed minor bug where if you were going to Generate Degree Wheel Readings,

the program would not accept a blank for the starting deg increment to be

interpretted as 0.

Fixed bug where the graph type of "Cam Journals and Lobes" was not

graphing the "Actual Cam Profile" (corrected for the radius of the

measurement probe) but the tappet lift profile.

Fixed bug where the graph types of "Cam Journals" was not being done

correctly, especially if the profile and journals were measured in, say,

English inches and then converted to Metric mm (or vice versa).

Fixed bug where the graph types which included Absolute Lift could get some

incorrect conversions because a flag (ConvertUnits% = 0) was not always

being reset.

Added new Preference called "Do v3.8 Ono Sokki Virtual Follower" to let

you have the program exactly duplicate the Virtual Follower simulation

done in v3.8.  Starting at v4.0 A.024, the default simulation is very

slightly from v3.8.

TDC Measurements from "Find TDC w Sensor" are now stored in debugging

file called:  TDCLogFile.txt

Fixed a bug where the rotary sensor's index could get corrupted if you

had used the feature of "Find TDC w Sensor".

--------------- Jan 4, 2015 Version 4.0 A.021  --------------------

Fixed bug where a lable in the 'Test/Cam Setup' screen which said

"Click Here...." did not produce the info message when you clicked

on it.

Fixed bug where if you started to measure a cam which had some

'extra' advance or retard added to it from a setting in the

'Test/Cam Setup' screen was not zeroed out.  Now this is zeroed

out so the measured lobe has zero advance/retard when it is measured.

--------------- Nov 25, 2014 Version 4.0 A.020  --------------------

Fixed minor bug where if you changed the Graph Name for the first

file in the History Log, it would not stay permanent.

Program now accepts data frames from Cam Pro Plus (CPP) electronics

to as small as 4 bytes.  This should make it more compatible with

a wider range of Cam Pro Plus electronics.

--------------- Nov 15, 2014 Version 4.0 A.019  --------------------

Fixed bug where Cam Pro Plus version would not read correctly if

Linear sensor was not hooked up.

Program now hides some input which are not used for some particular

follower types in the Virtual Follower screen.

Changed some names and their help definitions of inputs in the

Virtual Follower screen.

--------------- Sept 12, 2014 Version 4.0 A.017  --------------------

Program now blanks out the flag for "Converted from Hand Measurements"

when you start a new test.

Program now lets you try to reverse this Conversion process.

Program now better explains you can 'Click here' for info

on why certain Test/Cam Setup settings are disabled.

--------------- June 24, 2014 Version 4.0 A.016  --------------------

Adjusted the "Hemi 99" layout for more correct lifter bore angles.

Electronics recording screen now will adjust so it will appear

even if the last saved Top and Left positions of the screen

would put it off the screen.

Made some refinements to process of measuring a cam with

the Rotary Encoder installed on something which spins

with the engine, but not cam or crankshaft.

Program will now keep a rotary sensor calibtation from the

Rotary Encoder installed on something which spins

with the engine, so it can be use for checking other

cylinders and check timing between lobes.

Program now keeps track of when you have "calibrated" the rotary

encoder when it is installed on something which spins with the

engine (not cam or crank).  It will warn the user if this

calibration was done quite a while ago and may have changed.

In Graph Screen, when you click on View, then Cam Summaries,

program now shows the cam data like CLine, Dur, RAR, etc.

Added option to Export Manufacturing Type file in IGES or IGS

format.

--------------- Mar 5, 2014 Version 4.0 A.015  --------------------

Program now warns if you are trying to do a graph or report which

requires the Virtual Follower analysis, but the test file has

not been identified as using a 'Virtual Follower'.

Fixed bug where the Cam Degrees were not being figured directly

if you chose the option for the Rotary Encoder to be mounted

on the Crankshaft.

Fixed bug where if you tried to find TDC again when Measuring

with Electronics, the program would not let you.

Made the notice about lift being either measured directly or

simulated by Virtual Follower a title of the graph on the main

screen to make it more apparent to the user what was being done.

--------------- Feb 8, 2014 Version 4.0 A.014  --------------------

Program now blanks out all SpringNumLabel$() before assigning

them.  Prior to this, if a cam had only 1 lobe, label 2 for lobe 2

could be assigned even if lobe 2 did not exist.

--------------- Feb 1, 2014 Version 4.0 A.013  --------------------

Added new Cam Layout type for "Hemi 99" for race only Mopar

1999 Hemi block and heads.

Program now provides better explanation of steps for measuring

absolute lift, which includes journals and base circle diameters.

Program now better uses the Cam Grinder graph options under the

list of graph types to 'Pick' in the Graph Specs screen.

Program now checks if you export Polar Data from the Graph screen

to ensure you will get true base circle data points.

Updated the Help screens for Exporting Cam Dr and Export

Manufacturing data files.

Restored some features which were inadvertantly lost in versions

released in Nov and Dec 2013.  These features related to exporting

data files and graphing, especially for the more advanced

Cam Grinder version.

Program now informs you if you export "Raw 'As Measured' Data" it

is always in inches, even if the program is set to use Millimeters.

--------------- Jan 11, 2014 Version 4.0 A.012  --------------------

Fixed some bugs introduces in v4.0 A.011 when a section of code

was put backk to an earlier version by mistake.  That section of

code has now been restored.

--------------- Jan 4, 2014 Version 4.0 A.011  --------------------

Eliminated an error message when creating reports for files created by

'Measure with Electronics' after being converted to 'Measured by hand'.

Program now displays a message if you Edit the duration but the cam

centerlines have been corrupted or are missing.  This would cause

the cam timing to change also with this editting.

--------------- Jan 2, 2014 Version 4.0 A.010  --------------------

Added more explanation about sensor motion needed for finding TDC.

During the Analyze feature, where 2 lobes were adjusted (advanced/retarded)

to find the best agreement between them, previously the adjustment was done

to 0.2 deg crank increments.  Now it is done in 0.1 degree increments.

Program now checks that the Probe Radius is a reasonable value before

doing any Virtual Follower calculations.

Improved the Sub AssignMaxLifts so that previews would be more accurate

when a cam could contain something different more than 1 intake and 1

exhaust lobe.

Added feature to increase or reduce the lobe durations by a percentage.

Cam Grinder Version only.

Program now gives an explanation of the note in the Test/Cam Setup

screen about the cam file being converted from various types of cam

files, which may disable certain 'Electronics' options.

--------------- Oct 30, 2013 Version 4.0 A.008  --------------------

Centered the company logo pic better from top to bottom on the

main screen if it was short and wide.

Program now does NOT bring up the recording screen when pressing

the F5 key unless you've set up the program to have Electronics

working.

Added variable PolarAdvance! so the amount of Advance or Retard

specified in the Test/Cam Setup would also be included in a special

Cam Grinder graph type of "View from End of Cam" = "Yes".

Fixed bug where program now recovers better if the Company Logo graphics

file is not available or has become corrupted.

Fixed a bug introduced in v4.1 A.001 where the Cam Grinder options

could produce strange looking accel graphs with Valve Lift very close

to zero.

--------------- May 11, 2013 Version 4.0 A.006  --------------------

Changes wording in Camshaft Layout Details from "Number of Intake Lobes"

to "Intake Lobes per Cylinder" to better explain what is needed.

Fixed bug where not all of the "More Options" features were activated

in the Test/Cam Setup screen, for Plus version.

For Cam Grinder version, if you convert a file format to 'Measured with

Electronics' so you can do more detailed analysis, the program will now

ask you if you want a Base Circle measurement included with the lobe

measurements.

Program now shows the proper Probe description when  Virtual Follower

calculations are being performed.  The correct probe WAS always being

used in the calculations, but the screen did not show this.

Got the Export options on the Graph Screen to work better.

Made the program less fussy about what it would call "turning the cam too

fast" when measuring with electronics.

--------------- Feb 18, 2013 Version 4.0 A.005  --------------------

Better ensure that the scale on the X axis for graphing cam profile

when viewed from "End of Cam" stays with 3 decimal places.  This

was not always the case if the graph was resized.

Added DoingPolarGraphMode% so program would allow smaller increments

for manually scaling the graph when X axis is small for graphing

the end view of the cam.

If you were quitting program and pressed 'Cancel' when asked to save

your changes, the program continued to shut down.  Now it stays open.

Fixed some bugs in the Prefrences screen where the 'Send Test Email'

button was not working.

Added more detail to the Help screen for Preferences, especially

for emailing.

--------------- jan 23, 2013 Version 4.0 A.004  --------------------

Added 'Cam Grinder' to About screen if Cam Grinder version.

Fixed bug where base circle was not added correctly to the lift

if it was not measured, but "simulated" from Virtual Follower Base

Circle input.

Changed how program was displaying cam profile when viewed from

"End of Cam".  Now it has bee been rotated 90 deg, so typical

cam timing shows intake lobe at approximately 10 o'clock and

exhaust at 2 o'clock positions.

--------------- Oct 1, 2012 Version 4.0 A.003  --------------------

Fixed bug where unlocking codes did not always work.

Added better explanation of Find TDC option on Main Screen for finding

TDC when Measuring with Electronics.

Program now checks for Cam Pro Plus electronics when you click on the Find

button in Tester Settings screen.

Program now states you need a 'straight through' serial cable for

the Cam Pro Plus electronics.

Now only look for Com Ports up to 15.  (16 can give problems).

Removed 'Preliminary Version' label on opening Splash Screen.

--------------- June 18, 2012 Version 4.0 A.001  --------------------

Here's the text of Appendix 6 in the user's manual, which covers

the v4.0's new features in more details with lots of figures.

Cam Analyzer has had many updates since this user manual was written

for the original v2.0 for Windows. These include 3.2A (Appendix 3),

3.2B (Appendix 4), v3.8 (Appendix 5) and now v4.0 (Appendix 6).

Here is a listing of some of the new features for Version 4.0:

Cam Analyzer v4.0 actually has 5 different versions now, which include:

Cam Analyzer Basic (for analyzing cam data from either manual

data entry or computer cam files)

Cam Analyzer Plus  (for more detailed analyzing of cam data

from either manual data entry or computer cam files)

Cam Analyzer Basic for use with the electronic Cam Test Stand

(CTS) sensors

Cam Analyzer Plus for use with the electronic Cam Test Stand

sensors

Cam Analyzer Plus for use with the electronic Cam Test Stand

sensors, with advanced Cam Grinder features

Note that some of these new features apply only to the Plus Version

and/or Cam Grinder version of the software.  Also note that the

Cam Grinder version contains all Plus Version features.

New Features for the Cam Grinder Version Only:

The Cam Grinder version of the program now allows you to measure each

lobe and NOT adjust for lifter bore angles and firing order.  Instead

of all Intake and Exhaust lobes lining up on top of each other (except

for manufacturing variations), the lobes fall out as they would as you

look at the end of the cam.  This is how most Cam Grinders prefer to

see the results displayed.  Fig A44B.

The Cam Grinder version of the program now allows you to measure the

absolute lift of the lobes and bearings.  This way you can get a picture

of the cam as viewed from the end, with all lobes shown as they are

ground on the cam.  You also can see how the lobes compare to the

journals and how the journals look compared to each other (cam bent).

In doing this, the program first asks you to measure the journals lying

on the V blocks, or first and last journal on cam if mounting the cam

on centers.   This you do with a dial indicator.  Then the program steps

you through a sequence of measurements on these journals on the stand.

This way any slight slope of the cam on the stand is corrected for.

After that, you must be careful to not adjust the height of the linear

encoder as this will cause errors in height measurements.  This way also

allows the CTS (Cam Test Stand) to measure the base circle of each lobe.

Figs A45, A46.

The Cam Grinder version of the program now allows you to pick various

data types to graph and combine them on 1 graph.  It also includes

several new data types for graphing (Figs A47-52), like:

Absolute Lift (This is lift measured from the center line of

the cam.  If you have a 1.100 diameter base circle and a .400

max tappet lift, you should see approximately .550 of lift on

base circle and .950 (.550 + .400) of lift at max lift.)

Figs A47, A48, A49.

Thrust Angle  (For a roller cam, this is the angle the contact

force is acting on the follower, typically putting some amount

of side loading or thrust on the follower.)

Contact Point (How far from center of the follower has the

contact point moved.)  Fig A50.

Radius of Curvature (The radius of curvature at this particular

point on the Actual Cam Profile.)

Raw Cam Data as Measured (This is what the linear encoder measured.)

Fig A49.

Actual Cam Profile (This is what a knife edge pointer would have

measured.  This is the Raw Cam Data as Measured but correcting

for the radius of the pointer which was doing the measurements.

This should be what a cam grinder would have for a design file.)

Fig A47.

Lift Frequency Analysis *

Acceleration Frequency Analysis (Fig A52). *

Jerk Frequency Analysis *

* These Frequency Analysis graphs can be done either vs Order Number or

RPM Based on the Natural Frequency of a valve spring.  The specs for the

Natural Frequency of the Valve Spring are contained in the Test/Cam Setup

Screen. See Appendix 7 for more details on this Frequency Analysis,

FFT Analysis.

The Cam Grinder version of the program now allows you to import X and Y

data or Polar coordinates of degrees and radius to generate a cam profile.

This is done by clicking on the Tappet Lift title column.  Then the Edit

Test Data screen opens up and there are new Advanced Import Features with

several options.  Figs A53, A54.

The Cam Grinder version of the program lets you enter valve spring info

to estimate the springs natural frequency.   This allows Frequency

Analysis graphs to be graphed vs Engine RPM rather frequency order

number.  Fig A52 and Appendix 7.

Program has an "Export Manufacturing Style Cam File".  Two basic formats

are currently available, P File and X,Y Data.  Fig A55.

Program has an Export Graph Data as File option, so you can export any

data you can graph as a comma separated text (ASCII) data file.  If you

click on this option, the program will ask for a file name.  If you give

it a .csv extension, it will open directly in Microsoft Excel.  Fig A50.

Program has an Edit option of Cut and Append.  Say you have recorded

360 degrees of data (typical for a Measured with Electronics file),

but the data starts right at the opening ramp.  You would like to have

the data start about 100 degrees before the opening ramp.  You could use

this command to take copy the last 100 data points and put them before

the opening ramp.  When done, it would appear that you started recording

the data 100 deg earlier.  (Typically this is only needed for fixing

mistakes, or working with VERY unusual cams like the Ducati desmodromic

cam.)

You can now select for a variable amount of Filtering (smoothing) the cam

lift data when exporting cam files (None, Some, Medium, Heavy).

(This is not allowed for cam files exporting Raw data.)

Major New Features for v4.0 Plus Version (also in Cam Grinder Version):

The program can now use most any radius pointer for measuring a cam for

doing the Virtual Follower analysis.  This means that you could measure

a cam with the .750" diameter Universal Roller and then correct to what

it would be with a .800", or .700" diameter roller.  You can also correct

to something really different, like the exact cam profile as ground,

like you were measuring the cam with a "knife edge" pointer.  An

additional advantage of using a follower instead of the encoder probe

directly is no side loading is put on the encoder probe, thus avoiding

possible damage.   Figs A44A, A56.

The Virtual Follower screen can now be enlarged for analysis in greater

detail.  Fig A56.

You now have 2 options for Analyze option at the graph screen:  Just

show 'the max difference between lobes, or show the difference between

the lobes at all data points.

Now the program draws the actual profile lift (like the profile was

measured with a knife edge pointer) on the Virtual Follower screen,

in addition to follower lift.  The program draws this in green and

made the text boxes green which relate to this measurement.  Fig A56.

Cam Card now has options for printing, printer setup, including a lobe

from a different file for either an intake or exhaust lobe if the cam you

are currently reporting has only Intake or Exhaust lobes, or including

the Company Logo graphics file on the card.  Fig A57.

Program now has a Filtering option to quickly find files fitting

certain criteria.  This would be like finding all files which have

CHEV in the file name or the comments section, or files having the

number 346 in the Grind number recorded after a certain date, etc.

Fig A56 and Appendix 8.

Seating Velocity report now includes option to estimate the lash

point on cams.  Fig A59.

The Custom Duration reports now have an option for including asymmetry.

Major New Features for v4.0 Std Version (also in Plus and Cam Grinder

Versions):

New emailing options have been added in Preferences to work better

with most any emailing system, like gmail, yahoo, etc. .  Fig A60.

Program now has the ability to import just 1 lobe from a single lobe

data file, like S96 or Comp Cams format into an existing Cam File.

This allows you to build a cam consisting of and Intake and Exhaust

lobe where the lobes can come from 2 different sources (different

files and different file formats).   Fig A91.

When opening files, a new option has been added of "List by Date

File was Last Saved".  This lets you more quickly find files you

have recently changed or worked with.

Program will now allow direct conversion from most any type of data

to 'Measured with Electronics' in a 2 step process.  This conversion

is necessary to allow you to do more advanced analysis of cam data.

Minor Changes/Bug Fixes for v4.0:

Added a Preference under the Calculations tab so you can change the

lash the program assumes for doing Valve Lift, Acceleration, Velocity,

etc graphs and reports.  Note that the Cam Card always assumes .006

inches for doing Advertised duration and events.

You can now select 'Some' for 'Lift Filtering' for making a Graph or

Report for most all cam files EXCEPT those Measured with Electronics

or Created from Simple Specs. This was what was done prior to v4.0.

Now you have an additional option of 'Some (including meas. w elec.)'

so that you can do further smoothing of the recorded lift data.  This

can be especially useful when creating certain special graphs, like

Simulating the Actual Cam Profile when it was measured with the pointer

or a roller of a certain diameter.

Fixed bug where conversion to 'Measured by Hand' was not done correctly

if you asked for it almost immediately after opening up the program.

Program now keeps ALL cam file data points imported from Cam Dr,

Cam Pro Plus, etc for Plus or Cam Grinder versions.  Limit for std

version is still data points with .0015 lift or greater.

Program now correctly keeps cam Centerlines, Advance, etc when you

import Cam Dr files when you ask to keep ALL timing events.

Printed graphs have been improved.  Previously there could be a

broken border on the left side.

Fixed some bugs where doing graphs where extra lobes could be included

in a graph which were not requested.

Fixed a bug where a graph may not be updated correctly if you did not

go back through the Graph Specs screen.

Program now does not do additional smoothing to lift curve if data

is from a computer generated file.  This is to save considerable

calculation time.

Company Logo now appears on the main screen.  Plus and Cam Grinder

Version Only.  Fig A44B.

Printouts have been refined to look better with various screen

resolutions and operating systems.

The program now better imports certain data files which may have

been stored in different units than what the program is set up

for, inches vs mm.

Fixed bug where Intro Help Messages would cause the first graph

open when starting program to possibly be bad.

Added more Cam Layout templates, 240/300 Ford 6 cylinder, 170/200

Ford 6 cylinder, 250/292 Chevy 6 cylinder.

The program now looks for lift points greater than 10 inches when

reading in Cam Dr, Cam Pro Plus, etc cam files and edits them out.

This can occasionally happen and is likely caused from the imported

file being slightly corrupted, or being a slightly different version

than we have encountered before.

Fixed a minor bug where the graph on the main screen may not

completely show all lobes.

Fixed a bug where the graph screen could be significantly more

narrow than what was available on the screen.

In the Plus and Cam Grinder versions, now you can Advance or Retard

the cam degree wheel readings by up to 360 degrees under the Edit

options.  If you do this command more than once, you can advance or

retard readings even more.

^^^^^^^^^ Version 4.0 (June 2012) Changes are Above this point ^^^^^^^^^

--------------- April 9, 2011 Version 3.8 A.031  --------------------

Left in some new, non-finalized features by mistake.  This messed up

the Virtual Follower calculations.

Reduced max Crown Radius for Virtual Follower Flat tappet to level which

does not produce errors.

Fixed some items for Editting manual data entry, specifically being

able to delete an individual row for more data and deleting an

individual row.

Program now automatically advances the text input box for manual data entry

of cam deg and lift data much better if you are entering data beyond the

standard lifts generated.

Fixed a persistant bug which was believed to be completely fixed in

v3.8 A.030 where some data points could be removed when the screen was

continually resized and maximized.

Fixed some bugs in Seating Velocity calculations.

For Seating Velocity calculations, now the program lets you specify the

Filtering level to apply to the velocity data for determining the

cam velocity for the calculations.

--------------- April 4, 2011 Version 3.8 A.030  --------------------

When converting a file to "Measured with Electronics", the program

now keeps the minimum lift as the lowest lift measured, which is

NOT the default typically used by the program.  If you

click on the 'More Options' button at the bottom of the Test Options

screen, you will see this option.

(Previously, the AVERAGE lift on base circle, not lowest lift, was

used.  With large base circle runout, this could be a problem.)

Program now lets you keep the centerlines of a Cam Dr file.  This is

done by setting "Keep" to "Lobe Sep. and Int/Exh CL".

Now graphs on  the Main Screen draw the DEG scales better, so negative

crank degrees are shown correctly for even degree increments

When manually entering Crank Degrees for 'Measured by Hand' cam profiles,

the program would not allow for some very large duration cams.

Fixed bug where if you were entering lift data manually, and changed the

lobe (clicked on a different lobe tab) before completely entering the

number, some data could be erased.

When measuring In Engine, the program should now not continue to ask

if you want to Find TDC after you have found it for the first lobe.

--------------- Feb 11, 2011 Version 3.8 A.029  --------------------

Electronics Readings screen now "flashes" black in addition to beeping

when you have completed 400 deg rotation for measuring a lobe with

electronics.

Fixed some bugs concerning Exporting of Cam Files.

Program now also looks for Eng An Pro v3.9 automatically for exporting,

and will include Win 7 and Vista installations.

Fixed bug where the "Lash" for the Seating Velocity reports was being

added to the existing lash in the Test/Cam Setup screen, Plus Version

Only.  Now the existing lash in the Test/Cam Setup screen is NOT included

so the lash your enter for the Seating Velocity report is the entire

lash used.

Program and installer are now digitally signed, providing for less

warning messages, and less problems from anti-virus software.

--------------- Jan 16, 2011 Version 3.8 A.027  --------------------

Fixed bug where program only allowed Com Ports up to 8.  Now they go

up to Com 15.

Fixed a bug discovered when doing Harley Davidson cams with significant

uneven firing crankshaft.

Made program more forgiving for user finding TDC with 'TDC Bridge'

or on New Design stands when linear encoder can slide past V Blocks.

Improved the Graph Increments to follow even 180 degree increments

for very large ranges of degrees, and labeling them BDC and TDC.

--------------- July 6, 2010 Version 3.8 A.025  --------------------

Fixed bug where program would not export files to Engine Analyzer

Pro v3.9 correctly unless they were measured by the Cam Test Stand.

Program had increased the maximum height for duration and lobe area

reports up to .650 inches (13 mm) in v3.8 A.001, but somehow this

got eliminated in a later version.

Added 'Request Remote Assistance' feature under Help on main screen.

--------------- June 24, 2010 Version 3.8 A.024  --------------------

Fixed possible bug introduced in v3.8 A.023 where the electronics

may not record correctly.

--------------- May 26, 2010 Version 3.8 A.023  --------------------

Fixed minor bug where program would not always remember setting Com Port

to Port #1.  Other port #s worked fine.

Program now returns to the same lobe on the main screen even if you

make some major changes to the Cam or Virtual Follower specs in the

Test/Cam Setup screen.

Added new Edit option on main screen, to be able to Erase a lobe which

was measured with electronics.

Program now no longer shows the entire path of a file in the History Log

if it is stored on the default CamData folder.

Program should no longer warn about checking Encoder Options when you

start a new test when you are measuring the cam on the stand.

Program is now better at matching the exact desired centerline you

requested when measuring a cam with electronics and timing the cam

based on Centerline.

Fixed bug (probably introduced in v3.8 A.022) where certain Cam Card

data was not correct for cams measured with electronics and 1 intake

and 1 exh lobe per cylinder.

--------------- April 30, 2010 Version 3.8 A.022  --------------------

Fixed a rare bug where if you switch the cam data from 'Measured with

Electronics' to 'Measured by Hand', the program would now always find

the correct single Intake and single Exhaust Lobes to save if you had

multiple or 0 intake or exhaust lobes on a cam.

Fixed bug (probably introduced in v3.8 A.015) where Exh Lift at TDC may

not be correct for cam card report.

Fixed bug where Exh Gross Lift was not being updated when you were

working with simple cam specs, like describing cam by duration at .050".

Program now suggests saving to the 'My-Tests' folder if the folder you

opened a test from is not available (like the Examples folder).

Program now restores the correct Test File Name if you cancel out of opening

a Cam Dr or CPP file.

Program now displays more info in the Title Bar at the top of the program's

main screen when there is a very long test file name.

Program should now find Acrobat better (to display user's manual) for

various versions of Windows Vista and Win 7.

--------------- Nov 6, 2009 Version 3.8 A.019  --------------------

Fixed bug in Std version where printouts could have some overwritten

text of the Test Summary data.

Fixed bug where a possibly erroneous 'Asymetry' value could show up in

some reports in the Std version.

--------------- Nov 5, 2009 Version 3.8 A.018  --------------------

Program now allows file names in History up to 100 characters

(was only 30).

Fixed bug where if you loaded a Plus Version file which used

the where Method to Determine TDC' was set to 'User finds TDC

with Sensor' for finding TDC, the program could "bomb" if you

went into the Test/Cam Setup specs screen.

Fixed bug where the Alternate Lifts for Duration reports were

not being saved in the Preferences screen.

--------------- Aug 8, 2009 Version 3.8 A.016  --------------------

Fixed bug where now S96 files can be exported for individual lobes

on cams with multiple intake and/or exhaust lobes.

--------------- July 11, 2009 Version 3.8 A.015  --------------------

Fixed bug where cam lift for duration reports for cams with just exh

lobes were not being displayed in the reports.  This could also affect

Cam Cards for these cams.

Fixed bug where not all data for Cam Cards were being displayed for

cams with more than 1 intake or exhaust lobe.

Cam Card now reports which lobe is being used for cams with multiple

intake and/or exhaust lobes.

--------------- June 19, 2009 Version 3.8 A.014  --------------------

Fixed bug believe to have been introduced in v3.8 A.012 where the

program did not correctly convert profiles measured with the probe

to the a virtual follower profile.

Fixed a bug where the input box for graphing a particular lobe (A,B,C)

on cams with 2 or more intake or exhaust lobes was not visible for you

to access.

--------------- June 4, 2009 Version 3.8 A.013  --------------------

Fixed bug where recent changes dropped the "Virtual OHC End Pivot

Rocker" follower type in the Virtual Follower specs by mistake.

--------------- May 27, 2009 Version 3.8 A.012  --------------------

Now when you get an error message saying you are turning the cam too

fast, the program tells you exactly where on the lobe (lift and

opening/closing ramp) the cam was turned too fast.

Fixed bug where unusual cam layouts would not correct apply changes

to the Virtual Follower specs screen to intake and exhaust lobes

correctly.

Fixed bug where you could not "Pick" lobes on a graph with a cylinder

number higher than the number of cylinders on the cam.  For example,

if you had a cam with cylinder numbers 1,3,5,7, you could not "Pick"

a cylinder number higher than 4.

Fixed bug where program would not graph or report All cylinders if

the cylinder numbering on the cam incremented by 2, like 1,3,5,7.

Now after fixing this bug, for these type of cams,  you must "Pick"

1,3,5,7 to graph All lobes.

--------------- May 20, 2009 Version 3.8 A.011  --------------------

Increased maximum number of characters allowed for test comments from

400 to 1400.

Fixed bug where program would not always subtract out lash when graphing

valve lift results.

Fixed bug where program was allowing some kinds of Edit options for cam

profiles measured with electronics.  These types of edits are not allowed

unless you convert these files to the "Type of Cam Data" as "Measured by

Hand" in Test/Cam Setup.

Put in extra checks and warnings when you delete a test file or a test

folder in the screen for "Open (from all saved tests)".

Program now asks which lobe on which to base the Cam Card if there are more

than 1 intake or exhaust lobe for cylinder #1.

Fixed bug where if you click on the name Rocker Ratio when Rocker Ratio was

being determined by Virtual Follower calculations (there was no field in

which to enter Rocker Ratio), the program would bomb out.

Fixed bug where the Lobe Tabs across the top of the main screen did

not match the lobes on the cam drawing on the bottom if the cylinders

incremented by 2 or more between lobes.

Fixed bug where the Open and Save features for the Virtual Follower inputs

were not being done.

Added routine to check if Vista VirtualStore files have been copied

back to normal Cam Analyzer folders.

Now program should require you to 'Allow' it to run in Vista (same as right click

on desktop icon, then select 'Run As Administrator').  This should make the program

more Vista compatible.

--------------- Feb 28, 2009 Version 3.8 A.009  --------------------

Improved the accuracy and reliability of Exporting S96 cam files.

Added new Preference to allow program to eliminate occasional Noise Spikes

in recorded data.

Changed the Options in the Data Recording Screen to now include a

"Troubleshooting" section with some new Troubleshooting features.

Added much better Help information for the Data Recording Screen.

--------------- Aug 15, 2008 Version 3.8 A.007  --------------------

Fixed bug where program would not offer the Preference to "Show More

Options" button for the Plus version when measuring the cam with

Electronics.

--------------- July 14, 2008 Version 3.8 A.006  --------------------

When graphing Cam Data for a Virtual Follower Rocker Arm system, the

program will now graph the Raw Cam Data recorded with the pointer

directly on the lobe.

Fixed a bug where if you graphed Valve Data for a Virtual Follower

Rocker Arm system, the data was multiplied by the (hidden) Rocker

Ratio and was wrong.

Added "Stop Showing Help Tips" and "Restore Defaults" buttons

to the Preferences screen.

Fixed some bugs where some choices were not set correctly in the EZ Start

Wizard if you choose 'Yes' to run another test just like the current test.

--------------- July 9, 2008 Version 3.8 A.003  --------------------

Program now opens the library automatically from the EZ Start Wizard

if you choose to pick a test for a pattern.

Updated Preference settings for being better defaults.

Erased History Log for initial program installation.

Program no longer tries to copy config info when copying files from

an old Cam Analyzer program.

The 'Unlock Plus Features' is now always displayed.

Refreshed the initial SB Chevy on Stand file so the virtual follower

specs would work well with the Plus Demo.

Eliminated 'Preliminary Version' from intro splash screen.

Fixed bug in EZ Start Wizard where it could bomb program depending

on your choices.

EZ Start Wizard now knows what Measurement Type you will do even if

you don't select it in the program.

The Watch Cam/Follower button now is always shown with the Plus Version

and if you click on it, the program now explains why you may not

be able to Watch Cam/Follower if you are not using Virtual Follower.

***************  July 1, 2008 Version 3.8 A.001  ****************

Numbers in (), like (Fig A22), mean Fig A22 in the user's manual.

The full user's manual or the v3.8 Supplement is available by clicking

on Help, then selecting User's Manual or just v3.8 Supplement.

Cam Analyzer has had many updates since this user manual was written

for the original v2.0 for Windows. These include 3.2A (Appendix 3),

3.2B (Appendix 4) and now v3.8 (Appendix 5).  Here is a brief listing

of some of the features new since v3.2B was released, including Version

3.8s new features.

Cam Analyzer v3.8 actually has 4 different versions now, which include:

Cam Analyzer Basic

Cam Analyzer Plus

Cam Analyzer Basic with features to use the electronic Cam Test Stand sensors

Cam Analyzer Plus with features to use the electronic Cam Test Stand sensors

Prior to this, all versions communicated to the Cam Test Stand, however the Plus

version added very little to the user who did not have the Test Stand.  Now the

Plus version does add several features for users not using the Cam Test Stand.

Note that some of these new features apply only to the Plus Version of the software.

New Features for General Program Operation:

The program now has an EZ Start Wizard to walk beginners through the steps of

starting a new test.  (Fig A20)  This feature should greatly aid the user to

understand all options which can affect the various tests they can run.

Added several features for emailing graphs, files and PDF reports (if you

have the full Adobe Acrobat  or some other software which offers a PDF

printer option).  (Fig A21)

Click on File at top main screen and you will see new Import and

Export options.  The Export command is similar to the old Create

Cam Dr or Comp Cams files.  The new v3.8 also includes creating

S96 format files, raw data files for data from cams Measured

with Electronics, exporting raw data at 1 crank degree (0.5 cam

degree) intervals, and default location for the Engine Analyzer

Pro v3.5s Cam Files.  There are 2 raw data export options, Raw

As Measured data which is the measured data before any

conversions, and Raw Converted data is the profile after any

conversions (like virtual follower conversions) have been done.

The Import command has 3 options, mainly explaining how you can

open files of several different cam file formats, and one

discussed in the paragraph below.  (Fig A22)

Program will automatically look for old v3.2 files and copy them

over to this new v3.8.  You can also do this manually by clicking

on File at top main screen, then Import, then Import All Files

from a Cam Analyzer Program.  (Fig A22)

Program now has a 'Website' button in the About screen for

jumping to www.performancetrends.com website.

Program has added some new 'Edit' commands for copying cam

data to and from the Cam Analyzer to other analysis programs.

One is the ability to right click on the top of the data columns

in the lower left corner of the main screen, and select to

copy the Lift Only or Lift and Degree data to the Windows Clipboard.

More include to click on Edit at top of main screen, then:

Add To/Subtract From to add or subtract a user defined amount of lift.

Make Mirror Image

Make Centerlines Equal (zero advance

See Fig A23.

The Preferences screen has grown, so weve now redone the

layout to include tabs for the various groups of Preferences.

(Fig A24)

Now when you switch between Inches and Millimeters for the

Lift Increments in Preferences, the program also converts:

Specified lifts for the standard duration reports is

also converted to the new lift units.

Dimensions in the Cam/Test Setup screen.

Dimensions in the Virtual Follower screen.

Program now has added Backup and Restore commands under File

at the main screen.  (Fig A22)

The main screen now displays better for screen resolutions

greater than 600 x 800.

The screen colors are now more compatible with Windows XP.

Now when you click on a graphed point on the main screen,

the row for that data point gets highlighted in the table

of results.

Program now better remembers the printer type and printer

page orientation when you make changes to the Windows

Printer Setup.

Program now has new preference to let you pick a Company

Logo graphics file to appear in your printouts.  The logo

prints out in Graph printouts also.  (Fig A24)  Plus Version Only.

There have been several refinements to the program to

produce better looking printouts for graphs and reports.

Updated the routine to locate an appropriate version of

Adobe Acrobat  to view the PDF  files for the user manual.

A new Preference has been added to allow you to change

the line colors used for graphing and printing graph lines.

(Fig A24)

New Format option to Open/Save/Edit a particular graph

format.  This allows you to save the graph type, graph

scales, etc to a particular name.  Then you can easily

Open these saved formats to reproduce a graph format in

the future.  This is available by clicking on Format,

then Open/Save/Edit Graph Format.  (Fig A25)

The Graph screen now has a Format feature to display the

Legend (line labels) on the right side in a larger font,

and scroll up and down through the labels.  Since the font

is larger, not all labels can be displayed on the screen

if several data types are being graphed.  (Fig A26)

The Graph screen can now accommodate up to 72 individual

graph lines (data types).  (Fig A26)

Added several new features for writing ASCII files from reports.

(Fig A27)

Report names can now include the path.

Now you can browse to find or create a file name and path

for writing the ASCII file.

ASCII reports can now include the test comments.

Program now remembers ASCII file settings.

The Preference which allows you to choose a default Floppy/CD

drive has been expanded to include letter drives from A to Z.

Program now prints headers of graphs and reports the same,

and modifies the spacing to better ensure that long test

file names are printed completely.  As mentioned earlier,

you can also include a company logo graphic image in the

Plus Version.

Made graph line flash rate independent of computer speed

(.2 second flash rate).

New Outputs (Features for Graphs and Reports):

Note:  All these new features apply Only to the Plus Version of Cam

Analyzer.

You can now Advance and Retard the cam profile after it has

been Measured with Electronics.  This is a very powerful

feature for making adjustments and analysis after the cam has

been measured.  Plus Version Only.  (Fig A28)

You can convert most any cam data to the Cam Analyzer format

of Measured with Electronics.  This format has several of

additional analysis options available (like the several new

Report Types below), and also enables you to use the Virtual

Follower calculations.  This means that you can measure a cam

profile by hand with the dial indicator directly on the cam

lobe (no follower), convert it to this format, and then use

the Virtual Follower calculations to see what this cam would

do with various roller diameter followers, or OHC Rocker Arm

geometries.  This also means that you can import a cam profile

measured by some other system (Andrews, Cam Dr, Cam Pro Plus,

etc) with the encoder directly on the cam lobe (no follower),

and do the same Virtual Follower calculations.  Plus Version

Only.  (Fig A29)

A new calculated output has been added of Asymmetry.

Asymmetry is the measure of how much more aggressive the

intake opening profile is than the exhaust closing profile.

For a symmetric lobe, the opening and closing profiles are

the same (Asymmetry = 0).  If the opening profile is more

steep (aggressive), then the lobe may have, say, 4 degrees

of asymmetry at a particular lift, say .050 lift.  Technically

this means the centerline at .050 lift is retarded from the

centerline at the maximum lift point.  Plus Version Only.

(Fig A30)

Several new Report Types have been added for the Plus Version

Only, which include:

Lobe Analysis Report, similar to the Cam Profile

Evaluation Program reports in Don Hubbards

book Camshaft Reference Handbook  (Fig A31)

Custom Duration Report, which lets you specify up to

20 lifts at which to measure duration, opening

and closing events.  It also has options for

the lifts to be tappet lift (like the standard

duration report) or Valve Lift (correcting for

lash and rocker arm ratio), and whether to

include the new Asymmetry calculation. (Fig A32)

Seating Velocity Report, which calculates the valve closing

velocity at several valve lashes of your choosing

and 3 RPMs of your choosing.  Don Hubbards book

Camshaft Reference Handbook gives some rules

of thumb for acceptable seating velocities,

which are included in the program. (Fig A33)

Checking Limits Report, which lets you design several

checks to be made on the cam profile, like

duration/opening event/closing event at a

particular lift, maximum lift, centerline and

lobe separation.  This is a particularily useful

report if you are checking cams to meet some

quality control specs or to see if they are

legal for particular race classes. (Fig A34)

Added feature for doing Comparison Reports of one test versus

another.  (Fig A35)  Plus Version Only.

A new Graph menu option is Analyze.  At this time the only

option under this heading is Find Difference Between Lobes.

If you have graphed the same type lobes (intake or exhaust)

from 2 different tests, the program will find the matched

pairs of lobes, like Intake #2 for both tests.  It will then

go through the difference in lift of both lobes at every 0.1

degrees and save the largest difference.  Then it will advance

or retard the second lobe with respect to the first lobe and do

this comparison again until it finds the advance and retard

which produces the lowest maximum difference.  It will then

report this maximum difference for each lobe pair.  (Fig A36)

Plus Version Only.

A new Watch Cam/Follower screen lets you watch how the Virtual

Follower is interacting with the measured cam profile.  This new

screen shows the instantaneous lift and rocker ratio both at each

cam degree for the cam and now for the actual contact point on

the cam.  Data for the actual contact point shows more accurate

instantaneous rocker arm ratios.  (Fig A37)  Plus Version Only.

The Cam Card now includes duration at either .200 inches or 5

mm lift.  (Fig A38)  Plus Version Only.

New Features for Various Types of Cams, Cam Measurement Setups,

and Electronics:

Note:  Many new features in this section only apply to the

Cam Analyzer version which has had the Cam Test Stand

features activated.

Two new methods of indexing the rotary encoder have been added.

(Fig A39)  These new methods are used instead of you specifying

a particular event occurs at a particular degree for the #1

Intake Lobe, typically, say, 106 deg Centerline.  These methods

are:

User Finds TDC with Stops.

User Finds TDC with Sensor (typically for using the

Performance Trends TDC Bridge)  Plus Version Only.

For either method you will have a Set TDC <F4> option in the

electronics Recording screen.  You will click on the option or

press <F4> key and the program will direct you through a series

of steps to find TDC.

User Finds TDC with Stops is useful for measuring cams in the

engine with a piston stop, or where you can place the rotary

encoder exactly at TDC (for example by using the TDC mark on

the engine damper).  If you select this method, the program

will ask you to rotate the crank or cam (and therefore the

encoder as well) until the piston encounters the stop.  Then

it will ask you to rotate it the other way until it encounters

the stop again.  The program will split the difference and now

know exactly where TDC occurs.  If you can place the cam, crank

or encoder at exactly TDC, you will do that for the first stop,

and then not move the cam, crank or encoder at all and call it

the second point.  Splitting the difference of these 2 same

points will produce these same 2 points as TDC.

User Finds TDC with Sensor is typically used with the Performance

Trends TDC Bridge.  It lets you turn the dowel pin or keyway used

for timing the cam into a cam lobe.  The Bridge is now a flat tappet

follower.  You place the linear encoder on the bridge, press <F4>

and follow the program instructions.  It finds the centerline of

the dowel pin or key way and indexes the rotary encoder appropriately.

Plus Version Only

Notes:

Most dowel pins or key ways are NOT positioned at TDC, but some

degrees CW or CCW from TDC.  You will need to know that positioning

to use this feature.  For some popular cam designs, Performance

Trends has provided that timing in the Cam Layout Details screen.

Many aftermarket cams are NOT timed to the production timing of

the dowel pin or keyway.

The Cam Design Layout now has spots for entering the location of

the dowel pin or keyway from TDC in so many degrees, and whether

it is CW or CCW.  See point immediately above.   (Fig A40)

The program now has several new Cam Types preloaded, like the

427 SOHC Ford, more generic 4 and 6 cylinder engines, more GM

V-8s, etc.

The program has had several refinements for better and more

accurate operation with Millimeter lift increments, including:

Conversion of more data and settings from inches to

millimeters and vice versa when you change

the Millimeters/Inches Preference.

You can now create standard cam profiles from simple

inputs like Lift, Duration at 1 mm, etc using

Millimeters.

Virtual Follower features now work with Millimeter

dimensions and lift measurements.

The program can now better handle cams with just intake lobes,

just exhaust lobes, or most any combination of lobes of up to 3

lobes for each cylinder, intake or exhaust.  These options are

required for DOHC and other more modern engine designs.  Multiple

lobes for the particular cylinder and designated A, B and C, like

Int 1A, Int 1B, etc.  (Fig A41)

There is now a separate option for telling the program the cam you

are measuring in either on the Cam Test Stand or In the Engine.

The main difference are that if the cam is in the engine:

You do not need to know the Lifter Bore Angle Details.

That is because the engine block and/or heads

have the followers angled appropriately.

There is a good chance the linear encoder may extend when

you are measuring lift, which could be on the valve

itself or the follower.  On the stand the encoder

always retracts (is pushed in) as the follower is

lifted.

There is a chance the rotary encoder could be mounted on

the crankshaft or some other rotating component

instead of the camshaft.

The first option can be selected in the Test/Cam Setup screen.

If this is selected, then everything is done the same except the

Lifter Bore Angle Details are not used for timing when the lobes

for the different cylinders.  However, the program may warn you

that you should check the other 2 settings.

The second 2 options are available for setting in the Electronics

Recording screen.  If you select Rotary Encoder Mounting as

Other Rotating Component, then the program will ask you to

rotate the cam until it sees the cam lobe repeat a second time.

Then it will determine how many encoder rotations are equal to 1

cam rotation.  All these options are available in the Basic

Version except Other Rotating Component.  (Fig A40)

The Plus version allows you to set some More (advanced) Options

from the Test/Cam Setup screen.  (Fig A42)  These 3 options are:

Typical Cam Lobe (opens and closes once) - default

or   Cam Lobe may have 2 "bumps" (may open and close twice)

Just show data for Actual Lobe - default

or   Show All data measured (all 400 deg)

Zero Lift is Average Lift Measured on Base Circle - default

or   Zero Lift is Lowest Lift Measured on Base Circle

Typical Cam Lobe (opens and closes once) - default is a typical

cam lobe with one opening and one closing profile.  Cam Lobe may

have 2 "bumps" (may open and close twice) is used when the lobe

has 2 opening and closing profiles.  This typically is used for

Briggs and Stratton cams which have a small Easy Pull decompression

lobe occur during the compression stroke.

When you record a cam lobe with electronics, the Cam Analyzer

measures a full cam rotation plus 40 degrees more (360+40=400

cam degrees).  The extra 40 degrees is to check for measurement

accuracy.  The program then goes through all this data and tries

to find where the base circle stops and the opening and closing

ramps exactly start. Typically you are not interested in graphing

the base circle measurements because it takes up space on the graphs

and reports.  Then you would select Just show data for Actual

Lobe - default.  If you want to view all 400 degrees of measured

data, you would select Show All data measured (all 400 deg).

One case in particular is when you have selected the option Cam

Lobe may have 2 "bumps" (may open and close twice).

Base circle of the cam is not exactly zero lift.  There is some

small variation in the base circle lift and this is caused runout.

Up to now, the Cam Analyzer has used the AVERAGE base circle runout

as the zero lift setting.  Then you see small amounts of both positive

and negative lift on base circle.  For example if base circle runout

was .002 inches, you should see lifts from -.001 to +.001 inches.

That is the option of Zero Lift is Average Lift Measured on Base

Circle - default.  However, there are times you may want the very

lowest lift measured on base circle to be zero lift, or Zero Lift

is Lowest Lift Measured on Base Circle.  One case in particular is

when you have selected the option Cam Lobe may have 2 "bumps" (may

open and close twice).

Added new label to the Valve Train Layout, where you can pick whether

the cam rotates CW or CCW in the layout drawing.  If you say the cam

rotates CW in the drawing and the cam turns CW in the Cam Layout

Details, then a label appears that the view of the valve train layout

is from the Front of the engine.  If you say the cam rotates CCW in

the drawing and the cam turns CW in the Cam Layout Details, then a

label appears that the view of the valve train layout is from the

Rear of the engine.

Added new Clc button in the Valve Train Layout screen to allow

you to calculate the required of Angle and Distance for various

pivot points based on X and Y dimensions.

Added option to customize the calibration of the linear (lift)

sensor in the Tester Calibration screen.

The program will now work with some models of the Cam Pro Plus

electronics.

Program now does more extensive checking of valid com ports for

communicating to a FlowCom or Performance Trends Black Box and

Black Box II.  It checks up to Com Port 16, and gives better

suggestions at valid ports.  There is also a Find button in

the Test Calibration specs screen to find your electronics by

checking all available Com Ports. This feature is in the Basic

version also, but only with the Measure with Electronic Sensors

feature added.

^^^^^^^^^^^^^^  Version 3.8 Changes are Above this point  ^^^^^^^^^^^^

--------------- Jan 4, 2008 Version 3.2 B.091  --------------------

Fixed bug where the program would not "auto-advance" to the next lobe

if you had selected 'Cam Position' as the auto-advance method.

Fixed some problems when using "Find TDC with Sensor" like the TDC

Checking Bridge for finding the dowel pin or keyway location.

Fixed bug where option for Rotary Encoder on Crank was doubling the

degrees/rev.  It should have been cutting the degrees/rev in half.

--------------- Nov 10, 2007 Version 3.2 B.084  --------------------

Made graphs better keep the current scales and cursor line when printing.

Made graphs better expand when using the X axis scale expansion button.

Fixed bug where the Finding TDC from Sensor did not always work if cam was

turning CCW.

Fixed bug where if you did the Finding TDC from Sensor and then started

to measure a lobe other than #1 Intake, the program could produce cam

profiles which were off 180 degrees.

--------------- Oct 18, 2007 Version 3.2 B.082  --------------------

Program now eliminates blank rows in the reports, for a more

condensed screen and printouts.

Program now better positions vertical dividers in the printed reports.

Fixed bug where cam's measured on the Cam Test Stand, which did not start

with #1 cylinder, would not be reported correctly or have their lobes

identified correctly on graphs.

Fixed bug where the Adv Duration in the Cam Card was not using the

correct valve lash.

Fixed bug where program did not have correct firing order info if the

number of cylinders on a cam was less than the number of cylinders in

the firing order.

--------------- Sept 28, 2007 Version 3.2 B.079  --------------------

Program now better blanks out data for missing lobes in the Test

Summary section displayed in Reports.

Program now allows "User finds TDC with Sensor" in the Plus version.

Program did not "like" working with cam layouts for the CTS (cam test

stand) which did not start with #1 cylinder.  This problem fixed.

Added check to ensure you measured the first Intake lobe (or exhaust

lobe if only exhaust lobes on cam) before measuring any other lobes

with the Cam Test Stand.  This ensures the rotary encoder gets indexed

correctly before you measure other lobes.  This feature was working but

recent program mods seems to have disabled it.

Program was not keeping the 'Virtual Follower Setting' when you started

a new test and you were using a standard cam layout.

--------------- Aug 24, 2007 Version 3.2 B.078  --------------------

Added Sub FillInBlankVirtualFollowerType to assign a particular

Standard Follower Type (so NO virtual follower calculations are

done ) in the Virtual Follower screen, if no type has already been

assigned.

Added variable EliminateLargeXOffsets% to Sub Curvefit which improves

accuracy for 'Finding TDC with Sensor' when numbers get VERY large,

like the rotary encoder can produce.

Fixed problem where V-8, V-6, etc OHC cams were not indexed correctly

from #1 cylinder.

--------------- July 24, 2007 Version 3.2 B.077  --------------------

Made program more accomodating to linear encoders with higher resolution

than the standard Cam Test Stand encoder.

--------------- July 14, 2007 Version 3.2 B.076  --------------------

Fixed bug where a "Fix" introduced in v3.2 B.072 could blank out a Virtual

Follower profile after you watched in the animation screen.

Added option to customize the calibration of the linear (lift) sensor in

the Tester Calibration screen (Plus version only).

Comp Cams required a mod for a higher resolution linear encoder.

Added "Ford 427 SOHC..." Cam Design specs to the list of standard Cam Designs

supplied by Performance Trends.  NOTE:  The Lobe Separation obtained between Int

and Exh when measuring the cam on the Cam Test Stand is VERY sensitive to the

dimensions in the Plus Version Details valve train layout.

Added new label to the Valve Train Layout, where you can pick whether the

cam rotates CW or CCW in the layout drawing.  If you say the cam rotates

CW in the drawing and the cam turns CW in the Cam Layout Details, then

a label appears that the view of the valve train layout is from the Front

of the engine.  If you say the cam rotates CCW in the drawing and the cam

turns CW in the Cam Layout Details, then a label appears that the view

of the valve train layout is from the Rear of the engine.

Fixed bug where the Plus Version Details were getting blanked out intermittently.

The Virtual Follower screen now shows the instantaneous lift and rocker ratio both

at each cam degree for the cam AND now for the actual Contact Point on the cam,

which produces much more realistic Instantaneous Rocker Arm Ratios.

Now program only allows Comparison Reports (comparing different cams from

different files) for certain types of data.

Program now does a better job of aligning data for Comparison Reports

for more accurate reporting of results.

Program does not allow for Comparison Reports for certain report types,

like Comparing Lobes, Cam Card, etc.

--------------- June 2, 2007 Version 3.2 B.072  --------------------

Fixed bug where a "Fix" introduced in v3.2 B.072 could blank out a Virtual

Follower profile after you watched in the animation screen.

Added option to customize the calibration of the linear (lift) sensor in

the Tester Calibration screen (Plus version only).

Added "Ford 427 SOHC..." Cam Design specs to the list of standard Cam Designs

supplied by Performance Trends.  NOTE:  The Lobe Separation obtained between Int

and Exh when measuring the cam on the Cam Test Stand is VERY sensitive to the

dimensions in the Plus Version Details valve train layout.  This is true of

all this particular style of valve train layout, where the rockers are on

opposite sides of the cam.

Added new label to the Valve Train Layout, where you can pick whether the

cam rotates CW or CCW in the layout drawing.  If you say the cam rotates

CW in the drawing and the cam turns CW in the Cam Layout Details, then

a label appears that the view of the valve train layout is from the Front

of the engine.  If you say the cam rotates CCW in the drawing and the cam

turns CW in the Cam Layout Details, then a label appears that the view

of the valve train layout is from the Rear of the engine.

Fixed bug where the Plus Version Details were getting blanked out intermittently.

The Virtual Follower screen now shows the instantaneous lift and rocker ratio both

at each cam degree for the cam AND now for the actual Contact Point on the cam,

which produces much more realistic Instantaneous Rocker Arm Ratios.

--------------- June 2, 2007 Version 3.2 B.072  --------------------

Fixed bug for unusal situation for manual data entry, when the number

of rows available for data entry have blanks for the degrees.

Fixed some bugs where the Virtual Follower would produce inaccurate

results if the valve train layout for the OHC Rocker Arm valve trains

was VERY unusual.

Program now identifies whether the rotary encoder is on the Crank or

'Other' in the camshaft layout drawing on the main screen.

Added 2 new Briggs Single Cylinder and 2 new Generic DOHC 4 Cylinder

standard Cam Designs to the list of pre-programmed cam designs to pick from.

--------------- Apr 19, 2007 Version 3.2 B.071  --------------------

Added new Edit option of Add to/Subtract from Lift to let you add or

subtract some amount to the lift measurements.

Program can now recover from very bad inputs for Virtual Follower inputs

for Rocker Arm type OHC valve trains.

--------------- Mar 29, 2007 Version 3.2 B.070  --------------------

Fixed bug where if a cam had more than 1 lobe/cyl and you were indexing the

cam based on anything other than Centerline, the rotation index (deg)

would get screwed up when you measured the 2nd lobe for the first cylinder.

Fixed bug where the Cam Timing Value was not being set or refreshed correctly

in the Starting New Test Screen.

Program now does duration and lobe area reports in 1 mm increments.

Default low lift preference for mm has been changed from 1.0 mm down to

.8 mm because now 1.0 mm is a standard lift increment.

Fixed bug where the 1 mm duration and events in the cam card were not correct,

but were the actual events at 2 mm lift instead.

--------------- Mar 2, 2007 Version 3.2 B.067  --------------------

Acceptable limits for Cam Timing Values have been expanded to allow

for lifts in MM and Exhaust Opening Events for cams with only Exhaust

lobes in both the New Test Screen and Test/Cam Setup Screen.

--------------- Mar 1, 2007 Version 3.2 B.066  --------------------

Fixed bug where if reports were made of more than 1 lobe and the cam had 1 Intake

or 1 Exhaust lobe only per cylinder (not Int & Exh), not all lobes were displayed.

Fixed bug where if you requested timing a cam based on Lift at TDC, or Opening

event at .050" or 1 mm, it would not work for cams with only Exhaust lobes.

Fixed bug where measurements made in Millimeters would not always allow for doing

Virtual Follower simulation.

Refined the accuracy of timing a cam based on Lift at TDC, or Opening

event at .050" or 1 mm.  Before this, these events or lifts could be off by the

amount of runout error.

--------------- Feb 27, 2007 Version 3.2 B.065  --------------------

When went to more than 1 cylinder, when you clicked on a graph, it may

not have highlighted the correct point in the table of numbers.

Tried again to get the Cam Card to display fully on lower resolution screens.

Cam Card now redraws the graphics when it is resized.

When you make changes in Virtual Follower specs, the program now shows

progress of converting all the lobes, so you know something is happening.

Fixed a bug when converting lobes after Virtual Follower specs have changed,

that the program did not always distinguish the difference between intake

and exhaust lobes.

After a Virtual Follower conversion, the program used to be left displaying

the last lobe on the Main Screen.  Now it stays on the same lobe as when

you started the changes.

Added new Option in the Electronics screen for the Rotary Encoder.  You can

specify that the encoder is mounted on the Cam, Crankshaft, or 'Other'.

For 'Other', the program will automatically determine how many degrees of

rotary encoder rotation is equal to 1 degree of camshaft rotation.  This

makes it easy to mount the rotary encoder on a pulley or back side of a

belt when measuring the cam when in the engine.

Also, now the program stores the Rotary Encoder and Linear Encoder options

settings with each individual file, so you will know what the settings were.

Added new Example Cam Layouts:  Ford 1.6/1.9L CVH OHC 4 cyl,

AMC V-8 and Pontiac V-8

Fixed bug where a Cam Dr file was not leaving enough room for all the

intake lobe to be written.

The CompareReport% flag was not being reset, and you would get CompareReport%

messages on the main screen when opening a new test file.

Program no longer changes the Cam Design from a standard one (like Small

Block Chevy) to Custom when you change the Base Circle Diameter or

Crown Radius (flat tappet) or Roller Diameter (roller tappet).

Program now does not allow or asks you if you want to use some of the Edit

commands with profiles measured with the Cam Test Stand.

Fixed bug where the Edit command of Round Off Lift would also multiply

the lift by a factor, changing the lift when it should have only been

changing the number of decimal places.

Fixed bug where Exh Lift at TDC was not shown on Cam Card.

Added Duration at .200" / 5 mm lift to cam card.

Fixed bug where if some error occurred on the Graph Screen, the program

could loose the data for the current test you were working on.

Fixed bug where if you asked to change the Windows Printer Setup in the

Graph screen, it would revert back to the Main Screen for the change,

then go back to the Graph screen.

At some time prior to this version, program would include the new

Engine Analyzer Pro v3.5 to its list of locations to look for saving

Cam Dr files for exporting.

Fixed bug where the 'Lobe to Use' for Creating a Cam Dr file was not listed

correctly if the cam only had a few lobes.

Program now gives a message with the version number when it finds the

Engine Analyzer Pro when creating a Cam Dr file.

Made several mods in Sub MakeOutput to handle reports with only exhaust lobes.

--------------- Nov 11, 2006 Version 3.2 B.059  --------------------

Fixed bug where Reports and Cam Dr and Comp Cams files were not done

correctly when there were Intake and Exhaust data requested, but there

was not both intake and exhaust lobes on the cam.

Fixed bugs where Int or Exh lobes could be graphed even if you were not

asking for them.

Shrunk down the cam card width slightly to better display on some computers.

When starting a New Test, the program no longer blanks out the cam specs

if you have selected to build a cam profile from simple inputs.

Program now knows if you have changed the Test/Cam Setup in the New Test

screen, and refreshes the New Test screen with the appropriate changes.

Fixed bug where new Operator names added in the New Test screen were

also being added to the list of Cam Grinds.

--------------- Nov 5, 2006 Version 3.2 B.057  --------------------

Program now makes more reliable checks to warn you if you have

likely lost the TDC index of the rotary encoder.

'Program now shows it is the 'Plus' version in Title Bar and About screen.

The files 'SB Chev on Stand' and 'SB Ford on Stand' now have the new Plus

features included in their data, of Min Flat Tappet Diameter and Runout.

Fixed bug where the Plus version's Cam Card had the wrong Adv Duration

and events for Hydraulic cams.

Fixed bug where cam adv/retard was reported wrong in some places if you built

a cam from .050, .040 or seat timing events.

Fixed bug where the Lift at TDC for the Exh was not always reported in

the Cam Card.

Fixed bug where Cam Adv or Retard would get blanked out in the Test Setup Screen

after you had picked a new value.

Because sound does not always work on all computers, the program now displayes a

"Backward" message when you rotate a cam backward while measuring it.

Fixed bug where Rocker Ratio and Lash were not always used correctly when

calculating Valve Lift for cams with more than 1 Int and/or more than 1 exh lobe.

Fixed bug where program did not graph all lobes correctly for cams with more

than 1 Int and/or more than 1 exh lobe.

--------------- Oct 5, 2006 Version 3.2 B.052  --------------------

When starting a New Test, and you have selected 'User Finds TDC' when using electronics

and you tell program that you want to use the existing TDC settings in the BBII,

now the program does NOT ask you again in the Recording screen.

Now program gives better explanations in messages when setting TDC in Recording screen.

--------------- Sept 28, 2006 Version 3.2 B.051  --------------------

Fixed a couple of bugs for overlaying graphs of lobes for cams with

either all intake or all exhaust lobes.

When no int or exh data is available, or no int or exh lobes are on the cam,

the program better makes sure that the appropriate int or exh data is set to 0

or blanked out in the Test Summary section of reports or in the Cam Card.

Made the Cam Card part of the 'Plus' version only, but allow the user to

see the cam card for the 'Plus Version' demo, but only for SB Chevy on Stand and

SB Ford on Stand files.

--------------- Sept 6, 2006 Version 3.2 B.050  --------------------

Added Preference to read Cam Pro Plus electronics for Plus Version Only.

Fixed bug where now, using the F8 key, the program will back all the way

up to #1 cylinder Intake, even if that is not the first lobe on the cam.

Found that the CW/CCW switch on CPP could really screw up the results.

Fixed bug where finding TDC in Record screen did not split the difference

correctly when using a Piston Stop.

Added Command3 with Cancel = True to Test Setup screen so Esc key would work.

Added several checks in Starting a New Test to fill in critical specs

with acceptable defaults if these specs are blank.

Do not allow Examples folder to be used when starting a New Test, default

to 'My-Tests' folder.

If you choose Cam Pro Plus as the type of Electronics, the program does

not let you enter the Rotary Encoder's pulses per rev.  The program

knows what it should be.

--------------- Aug 24, 2006 Version 3.2 B.047  --------------------

Added an new option of 'User Sets TDC' for indexing the cam to TDC

in the Test Setup screen, when using the Cam Test Stand.

Added menu feature in the Electronics screen to 'Set TDC' if you have

selected the option of 'User Sets TDC' for indexing the cam to TDC.

This command can be activated by pressing <F4>.  You can find TDC

by either setting the crank to TDC based on timing marks, or by

running the piston up against a stop near TDC, turning the crank

both directions.

--------------- Aug 22, 2006 Version 3.2 B.046  --------------------

Added Option in Cam Electronics screen to Rezero the Lift Sensor.

This is not necessary to do as the program finds the average runout on

base circle and treats that as true zero.  However, some people like

to be able to set a zero lift reading.

Added Option in Cam Electronics screen to Reverse the readings of the

Linear Sensor Readings.  This was necessary when measuring valve "lift"

(actually valve movement down) as lift.

Added new feature of displaying and printing a "Cam Card".  This is a

new Report type and can also be found under File, then Print Cam Card

at the main screen.

The program now accepts a much wider range of lifts in the Preferences

menu for reporting duration, opening and closing events when you do

a Duration report.

The program now switches these 6 Lifts in Preferences between inches

and mm when you select either inches or mm as the units for

measuring lift.

New default lifts for these points for MM are: .1, .15, .2, .5, .75 and

1 mm lift.

Higher lift increments for these events for MM are now done in 2 mm

steps (previously done in 5 mm steps).

New default lift for Adv Events for Cam Card for Hydraulic lifters

is .006 for inches and .15 mm.

--------------- Aug 1, 2006 Version 3.2 B.044  --------------------

Made drawing of End Pivot RA Layout more universal in the drawing.

Added 2.3L Ford OHC and Eagle Talon OHC to the list of std Cam Layouts.

These require the Virtual Follower simulation to be done correctly.

Fixed bug where there was an error in cam rotation in Virtual Follower

for OHC Rocker Arm layouts.

Program now makes better checks if you make changes that the profiles

measured with the CTS must be refigured, especially for Virtual Follower

features.

Fixed bug where Virtual Follower features of the program now

differentiate between the Int and Exh followers.  It also now draws

the correct layout before you animate.

Fixed bug where Advance/Retard was not shown correctly on the

graph screen for lobes measured with the Cam Test Stand.

Added Graph Menu Item of Analyze to find max difference in lift between

2 lobes for all lobes for the first 2 cams graphed.  This is for Plus

Version only.

Program now remembers the Printer Orientation, landscape or portrait.

Program now shows the exact same Test (Cam) Summary specs as what you enter

if you create a cam profile from .050 events (English units) or 1 mm lift

(Metric Units).

Added History Log, Single Test and Multiple Test menu options to Reports screen

for Plus version.  This lets you do comparison reports between cam files for

the same lobes.  This is still somewhat preliminary.

Fixed bug where the "Cylinders to Graph" in the Report Specs would not

always work as designed.

Fixed bug introduced in v3.2 B.042 where the first and last data point to

graph was not being set correctly.

Fixed bug where column titles would not be printed in reports correctly

if some columns were blank.

Added several features to get better looking printouts.

Added Runout and Minimum Required Flat Tappet Diameter (based on peak

velocity) to Compare Lobes Report for Plus Version.

Added variable RestoreCurrentTestSpecs% so that if you X out of

Starting a New Test, the program would restore all data as if you had

never tried to Start a New Test.  The program did this before, but only

if you click on "Cancel"

--------------- May 1, 2006 Version 3.2 B.040  --------------------

Fixed bug where the Lobe to Use for creating Cam Dr or Comp Cams file

would not allow you to pick the available lobes.

Fixed some bugs where program was not correctly picking which lobes

to graph for multi valve engines.

Fixed bug where some drop down lists in the New Test Screen were not

filled in correctly if you started a new test without a test already

opened up on the main screen.

Fixed bug where the summary data for lobes like Duration, Lift, etc,

were not updated when a file was converted from English to Metric and

vice versa.

--------------- Apr 20, 2006 Version 3.2 B.038  --------------------

Added 2 example "Quick Check" files for easy starting point "templates"

if you just want to quickly measure #1 cyl intake and exhaust lobed.

--------------- Apr 12, 2006 Version 3.2 B.037  --------------------

Changed program operation to NOT do filtering on the raw lift data if

you are creating that data from simple specs.

When converting from Metric to English and Vice Versa, program now

converts appropriate readings from MM to inches and vice versa for

Plus Version Details screen and Test Setup screen.

Program can now create cam lobe data from simple inputs for Metric

version.

Program can now do Virtual Follower for Metric version.

Linear sensor gauge in Recording screen is now drawn to a scale of 0-25

mm for metric.

Fixed bug where actual recorded data was restored by specifying "Actual

Flat" or "Actual Roller" as the follower type in the Plus Version

Details, the lift was always shown in inches, even though MM units

were selected.

--------------- Mar 4, 2006 Version 3.2 B.036  --------------------

Added several things to deal with cylinder numbers greater than 10.  This includes

buttons to jump between different tabs, go to first tab and go to last tab, because

program only shows up to 16 tabs to keep tabs from getting too crowded.

Added feature to allow for 'odd firing' for cylinders in Cam Layout.

Program now blanks out certain data if no Duration or Lift is available for that

lobe.

Fixed bug where virtual lifter was not correctly calculating lift.

Program now accomodates up to 12 columns in the output for printed reports.

Now program spaces then graph legend closer to accomodate up to 48 graph lines.

Increased the dimension of several graph legend arrays to accomodate number of

legends up to 72 graph lines.  Previously you could get errors if you tried to

put a cursor on the graph lines with more than 36 graph lines being drawn.

Fixed some bugs which could occur if the rotary encoder did many revolutions without

being rezeroed.

Fixed bug where program could give error message "You are turning the cam too slowly."

even if that was not the problem.

Increased size of the "GraphAdvance" array to prevent errors when advancing/retarding

different lobes on graph screen.

Fixed bug where an error message "FilterLobe Er = 9" could occur when measuring a lobe.

Not all lobes were being included in graphs for cams with more than 1 intake or 1 exhaust

lobe.

Added new input in Graph Type specs screen, of Multi-Valve Lobe to Graph.  This lets you

pick which lobe, either A, B or C or All when there is more than 1 intake or 1 exhaust

lobe.

Fixed bug where graphs for a cam with Only Exhaust lobes would not be graphed correctly.

Added Label to main screen to show if tappet is simulated with Virtual Follower or

measured using the Actual follower.

Changed the "Honda DOHC" Example Layout Info to "Honda 4Cyl B Series" which now includes

all 3 lobes for each cylinder.

--------------- Jan 24, 2006 Version 3.2 B.032  --------------------

Fixed bug where Main Screen was not showing everything if resolution went

above 1024 x 768.

Fixed bug where Compare Report was not displaying all information for all

lobes.

Fixed bug where Lobe Seperation was not being done correctly when features

were added for multiple lobes per cylinder.

Fixed bug where FilterEr = 9 would occur, because Port number would be

less than 0 and program could bomb.  This was usually caused when you were

just asking to do a report on Cyl #1 Only.

Fixed bug where making reports could get program to bomb.

Added Mopar V10 and several Honda V6 and 4Cyl layouts to standard Cam layouts

for use when using the Cam Stand for measurements.

Fixed some bugs where program would not allow you to enter cylinder numbers

for the cam layout that did not include Cyl #1 or that were 2 digits, like

cylinder #10.

Added 'Clc' button to Virtual Follower screen to allow calculating angle and

distance from X and Y measurements.

Fixed bug where the Virtual Follower screen had the 'Fix Error' always turned

on for drawing the layout.  Now the layout is drawn exactly as entered (with

the errors) and only fixes the errors if you click on Fix Error.

--------------- Dec 10, 2005 Version 3.2 B.029  --------------------

Fixed bug where the Std BB Chevy layout and lifter bore angle data was not

correct.

Added checks to allow some Virtual Follower specs to change without calling

the Cam Type "Custom".  Plus Version Only.

Added Watch Cam/Follower button on Main Screen to watch animated valve train.

Plus Version Only.

Program now does correction for radius of pointer when doing Virtual Follower

calculations.  Plus Version Only.

Program is now better at connecting measured points to fill in for missing

data as cam is rotated on test stand.

Program now needs 2 consecutive readings to show more than 400 deg to say

you have rotated cam enough for stopping the test.

Added label to Recording screen to better show what lobe you are on.

MM are now displayed and measured in the Recording screen.

Fixed bug where default Virtual Follower specs were not loaded correctly.

Plus Version Only.

Fixed bug where comments were not always displayed in the Preview when opening

a test file.

PlusVersionDemo% now tells user features will only be displayed if using one of

the "on Stand" examples.

Put in 'Tip' message about Plus Version Demo.

Fixed bugs where the Correction for the Radius of the Measurement Pointer

was causing problems for the 2 OHC valve trains.

Fixed bug where program could not analyze a measured profile if the cam was

turned too slowly.

Allow saving/opening Floppy/CD rom drives from A - Z, Set in Preferences.

Added Browse button for Cam Dr Files.

Added several features to deal with Cams with all intakes or exhaust, multi valve

engines.

Fixed bug where Main Screen was not showing everything if resolution went

above 1024 x 768.

--------------- Sept 28, 2005 Version 3.2 B.020  --------------------

Fixed bug where program did not find #1 cylinder correctly if you

specified a low "Number of Cylinders to Test" it the Test/Cam Setup

screen for using the Electronics, like just 2 or 3 cylinders.

Changed the "Beep" method used by the program so beep sounds are heard

on more different types of computers.

--------------- Sept 3, 2005 Version 3.2 B.019  --------------------

Added ability to pick which cylinder to use when writing a Cam Dr or

Comp Cam's format cam file.

--------------- Aug 29, 2005 Version 3.2 B.018  --------------------

Fixed a couple of bugs when opening files using the "Advanced" button.

Fixed a bug where program would give error message "can't find folder

"cacurdat.pti"

Improved communications with electronics on very fast computers.

Added diagnostice option in frmFlowCom to Display Relative Computer Speed.

Program now changes menu item from Record (F1) to Stop Recording (F2) in the

recording screen.

Added checks to be sure at least .01 of cam lift is recorded in Recording

Screen.  If not, program does not keep the recorded data.

Repositioned the fix to refresh the main screen to display all controls so that

it was also done when the program advanced to the next lobe automatically.

Fixed bug where Operator and Grind was not being saved in the New Test Screen.

Added Preference to allow adjusting or turning off the check for difference

between #1 cylinder lobe measurements and subsequent lobes being measured

with electronics:  "% Dif. from #1 Cyl to Flag"

--------------- Aug 18, 2005 Version 3.2 B.014  --------------------

Added variable OpenFromHistoryLogFlag% from PFA for better updating of History

Log where opening a file from the History Log

Copied routine form PFA for opening a file from the History log.

Added variable in graphing routine to keep track which Individual

Cylinders are to be graphed.

Added Find button for Com Ports in Encoder Specs screen to check ports 1-8

for Black Box communications.

Now when you right click on a folder to delete, rename or copy it, that

folder is now selected also.  Previously, you had to left click it to

select it, then right click it to have the options presented to you.

Program now checks Firing Order and Cam Layout for obvious errors in inputs.

Program now checks inputs for the Cam Layout Details or accurate entry.

Fixed a bug which did not open the file from the History log correctly, or

update the History Log when a file was opened.

Fixed a bug where if you picked individual lobes from Picked List in Graph

Specs screen or History Log, the correct lobes were not graphed.

Updated the Help files for new screens and inputs added for 'Measure w

Electronics'

Now when you right click on Folders in the 'Open from All Saved Tests', the

folder you right click on is highlighted and chosen for the action.

Fixed bug where controls on main screen could disappear when clicking

on other tabs.

Fixed bug introduced in previous update where EZ Data Entry did not work

Put back frmFlowComCam to allow for EZ Data Entry.

--------------- June 29, 2005 Version 3.2 B.011  --------------------

Program no longer does "lobe (lift) filtering" when measured with

electronics, to save time making graphs, reports, etc.

Fixed bug when program asked you to look for Acrobat Reader (tm)

on the web.

Fixed bug where Comments you entered on Main Screen would not

always be saved.

--------------- June 18, 2005 Version 3.2 B.010  --------------------

Now rotary encoder can rotate backward several revs before a "roll

over" error occurs.

Eliminated some 'debugging' error messages.

Added Resize32.ocx system file to installer.

Made numerous changes for Electronics.

New Test options for electronics.

Added Edit Option of Mirror Image.

Added Troubleshoot menu options on main screen for electronics COM Ports.

--------------- Feb 10, 2005 Version 3.2 B.007  --------------------

When clicking on the Graph on the main screen, depending on the screen

resolution, you may not be shown the row for the data point you

clicked on.

When you would have the program look for files 'frmSeek', the program

could bomb on XP computers when encountering some protected folders

in XP.  Fixed this bug.

Fixed bug where "Home made" S96 files don't like to read 0 in a line

for lift, but will read .000000.

Fixed bug where the screen for printing Ascii Files (in the report

screen) would disappear as soon as it was displayed.

Made the routine which displays PDF files compatible with "finding"

Adobe Acrobat programs up to version 20.

--------------- Dec 8, 2004 Version 3.2 B.006  --------------------

Added frmFlowCom screen to read black box for computerized cam

measurements.

Preferences menu option is changed to Settings, under which you can

choose Preferences or Tester Calibration.

Developed method to create a cam file with both intake and exhaust

lobes from 2 seperate cam files, like 2 S96 files.

1  Open the S96 file for the Intake Lobe per your specs.  Once opened

at the main screen, note the Deg Wheel reading for the first point,

intake opening.

2  Open the same or different type of file with only 1 lobe, like

another S96 file for the Exhaust Lobe per your specs.  Once opened

at the main screen, click on Test Options and select Type of Test

Data as 'Measured by Hand'.  Also select Deg Wheel Type as

'0-180-0' and be sure 'Deg Steps' is set to 2.  Then click on

'Back' to return to the Main Screen.

3  At Main Screen, click on Intake tab at top to display Intake Lobe

which should be blank.  Click on Generate Deg Wheel Readings and at

this screen enter the first Deg Wheel reading (intake opening) from

step 1 above.  Also set 'Estimated Duration' to 500 deg to be sure

there is enough.  Then click on OK and the Original Intake tappet

lifts should now be visible.  If they are not indexed correctly,

click on Deg Wheel at the top of the Deg Wheel Readings column

and choose the Advance or Retard option to shift the Deg Wheel

readings as necessary.  Since Advance and Retard are limited

to 30 degrees max, you may need to do this several times.

Advanced 'Save As' now shows a Save button instead of Open Button.

Fixed bug where History Log could become corrupted with commas ",".

Fixed bug where Cam Data was not always completely erased when you were

starting a new test.

Program can now better handle EXTREMELY big exhaust durations correctly

when generating Deg Wheel increments (exh which opens Before TDC).

Program now shows the correct cylinder # on Tabs at top of main screen.

Fixed bug where if using 0-180-0 Deg Wheel type could produce blanks

in the Deg Wheel column instead of 0.  The blanks could cause problems.

Allowed for 3 times to display:  ExplainCantEditLifts%

Now allow a generic 360 line file to be read as a Generic format,

Type = "I-?"

Added Preference for EZ Data Entry increment size.

Fixed a couple more XP colors on labels.

Made graph flash rate independant of computer speed.

--------------- Dec 20, 2002 Version 3.2 A.012  --------------------

Fixed bug where graph on main screen was not always displayed so you

could read the X axis numbers.

Screen colors are now more compatible (attractive) with Windows XP.

Fixed a bug where a file created from cam file (Cam Dr, Cam Pro Plus,

etc.) would not display the Print Options menu (produced an error)

from the Report Screen.

--------------- Nov 2, 2002 Version 3.2 A.009  --------------------

Enlarged EZ Data Entry screen slightly so you can read words on

buttons better.

Now says "Reg Version" if you click on mnuRegTo.

--------------- Sept 18, 2002 Version 3.2 A.007  --------------------

Added BTDC/ABDC etc notation to cursor readings in graph screen.

Added BTDC/ABDC etc notation to reports for opening and closing points.

Fixed bug where Degree Wheel readings were not being done in reports

if you selected to make reports at 2 deg increments.

Fixed bug where graph screen could show Cam Deg when the graphs are

always done in Crank Deg increments.

--------------- Sept 17, 2002 Version 3.2 A.006  --------------------

Activated the Swap feature in File, then Edit (copy/swap lobes)

Fixed bug where dividing lines were not being drawn in printed reports.

Added option to choose the Deg Wheel Steps for printing a worksheet

in the Print Worksheet screen.

Fixed bug where if clicked on Reg Name with unlocked version, it still

said Demo version.

Fixed bug which could occur if clicked on Format, then Edit Printed ...

in the graph screen was clicked on and program could stop.

--------------- Aug 12, 2002 Version 3.2 A.002  --------------------

You can now write (export) files in Comp Cams  format.  This lets

you create individual file lobes of intake and exhaust lobes so

you can mix lobes from different cam files.

User manual is now access file from within the program by clicking

on Help at top of the Main Screen, then Display Users Manual.

The program uses Adobe Acrobat  to display it in PDF format.

--------------- Aug 2, 2002 Version 3.2 A.001  ---------------------

Program is now 32 bit, which means:

- It can use long file names.

- It should be more compatible with USB printers and Windows XP

and 2000.

Larger graph screen.

You can now select the amount of smoothing the program should do

to manually entered cam lift data.  For example, data entered at more

than every 5 degrees should probably have less than the Std smoothing.

You can now select to manually enter the degree increments in most any

step size you want.  This allows you to concentrate the readings in

only the area you want (like around .050 lift and peak lift).  This is

done by selecting User Entered for Degree Steps in Test Setup.

Program now tries to automatically recognize if a file was recorded with

millimeters or inches lift.  It then converts it as necessary for the

current Preference setting of either millimeters or inches lift.

------  Nov 15, 2001  v2.0 A.016   -----------------------------------

Added several edit options at top of main screen.

Added preference%(10) for mm/inch lift inputs.

------  Oct 20, 2001  v2.0 A.015   -----------------------------------

Added more fixes for "funny" combinations of Lobe Seperation,

Cam Advance, Lobe Centerlines, etc.  These can be caused from

importing cam profiles from Cam Pro files.

------  June 26, 2001  v2.0 A.014   -----------------------------------

Eliminated references to Port Flow Analyzer in the help items on the

Main Screen.

Give more warnings if creating a Cam Dr file that file is based on

current cam measurements on main screen.

Added option to Copy To or Paste From the Clipboard the Lift Data

Column.  Click on Top Row in Lift Column "Tappet Lift" to see

this option.

Fixed a problem where unusual and wrong centerline and cam advance

numbers were generated by the program and loaded into the Test Options

screen.  These caused problems because problem would flag out these

numerous errors if you change the "Type of Cam Data".

------  Mar 9, 2001  v2.0 A.011   -----------------------------------

Added options to read different lobes in Cam Pro Plus files.

------  Feb 27, 2001  v2.0 A.010   -----------------------------------

Fixed bug where data read from CamProPlus files (and probably

other cam file formats) were not always read correctly.  You would

see lift, velocity, acceleration and jerk being interpreted as lower

than what it should have been.

------  Feb 17, 2001  v2.0 A.009   -----------------------------------

Fixed a bug where printed worksheets were not correct for Deg Steps

greater than 5 degrees (like 10, 15 or 20 degrees).

------  Feb 10, 2001  v2.0 A.008   -----------------------------------

Made Duration report more "forgiving" if the lifts specified for

duration and events were lower than the measured data (i.e.,

you want duration at .003" but started recording data at .005").

------  Feb 2, 2001  v2.0 A.007   -----------------------------------

Now program displays degree wheel and graph on Main Screen

better withn 800 x 600 screen resolution.

EZ data entry now estimates lift for 1st and 2nd readings (it

used to say not possible).

Fixed problem where Duration was not estimated in "Generate

Deg Wheel Rreadings" if you had specified 20 deg increments.

Fixed bug where 'Designed Lash' was enabled in Test Options

screen even though you were measuring a cam by hand or were

using a cam file.

Flash rate on Graph Screen is now more consistent between

computers of vary different processing speeds.

------  Jan 15, 2001  v2.0 A.006   -----------------------------------

Added Edit Command to Copy Lobes option under File command.

------  Dec 15, 2000  v2.0 A.005   -----------------------------------

Fixed a bug where several cam specs were not displayed if you

selected the Type of Cam Data in Test Options as 'Create from

.050: '' Specs', etc.

------  Sept 18, 2000  v2.0 A.004   -----------------------------------

Test Summary for graphs and reports now shows "Hyd" when

the cam profile type is specified as Hydraulic.  It previously

showed a Lash value.

------  July 13, 2000  v2.0 A.003   -----------------------------------

Added an End of File character to Cam Dr files written.

Some software not written by Performance Trends required

this for reading Cam Dr files.

Fixed bug where the Cam Dr file created could be influenced

(made incorrect) by settings in the Report Options screen.

If you click on the 'Point' column on the Main Screen to delete

some points (rows), the program now checks if you are deleting

any points with Tappet Lift data.  If so, the program tells you

this and asks if you really want to do this.

------  June 8,2000  v2.0 A.002   -----------------------------------

Program now reads the Lobe Seperation from Cam Dr files it reads.

Note that these Centerlines may not match the Centerlines determined

by the Cam Analyzer, so lobes may actually be shifted.

Fixed bug where if the Operator.pti file was not present, the program

bombed when opening Find TDC screen.